Mobile Speak and Mobile Magnifier

for Symbian/S60 Phones

User Manual

for v4.60

Published by:

Code Factory, S.L.
Moving accessibility forward!

Rambla d'Egara, 148, 2-2
08221 Terrassa (Barcelona)
Spain

E-mail: info@codefactory.es
Website: http://www.codefactory.es

Information in this document is subject to change without notice. No part of this publication may be reproduced or transmitted in any form, or through any electronic or mechanical means, for any purpose, without the expressed written permission of Code Factory.

Copyright © 2010 by Code Factory, S.L. All rights reserved.

Contents

1	Introduction to Mobile Speak	4

2	What’s New in 4.0	4

2.1	Cross Platform	4

2.2	Touch	5

2.3	Multi-Voice	5

2.4	Ready for the Next Generation	5

2.5	Packed with New Features	5

2.6	New features, improvements and fixes in v4.60	6

3	Installation on Symbian Devices	7

3.1	Prerequisites	7

3.2	Installation	8

3.3	Uninstalling	9

3.4	Nokia Built-In TTS	9

3.5	Software Updates	10

3.6	Upgrading from a Previous Version	10

4	How-to's	10

4.1	How Do I Make a Phone Call?	10

4.2	How Do I Access the Main Menu?	11

4.3	How Do I Send an SMS?	11

4.4	How Do I Manage My Contact List?	11

4.5	How Do I Manage Calendar Appointments?	12

4.6	How Do I Read Text Content?	13

4.7	How Do I Check the Battery and Signal Levels?	13

4.8	How Do I Lock and Unlock the Phone?	13

4.9	How Do I Access the Mobile Speak Control Panel?	14

4.10	How Do I Change the Stylus Mode? (Touch screen devices)	14

4.11	How Do I Mute Mobile Speak?	14

4.12	How Do I Stop Mobile Speak?	14

5	Licensing	15

5.1	Activation Manager	15

5.2	Getting a Trial License	15

5.3	Activating a Full License	16

6	Commands	16

6.1	Keyboard Help Mode	16

6.2	About Layouts	17

7	Configure Mobile Speak	17

7.1	Mobile Speak Configuration	18

7.2	MS Profiles	18

7.3	Configuration Structure	18

7.4	Speech Settings	18

7.5	Reading Settings	20

7.6	General Settings	22

7.7	Verbosity Settings	23

7.8	Braille Settings	25

7.9	Magnifier Settings	26

7.10	Maps Settings (only on phones supporting v3.03 or v3.04 of Ovi Maps)	30

7.11	Options under the Left Soft Key	32

7.12	TTS and Voices	35

8	Mobile Speak 4 Touch Interface	36

8.1	Keypad Mode	36

8.2	Joystick Mode	37

8.3	Quadrant Mode	37

8.4	Command List (Accessed from Quadrant Mode)	39

8.5	Virtual Touch Keyboard (Accessed from Quadrant Mode)	39

8.6	Review Cursor Mode	41

8.7	Stylus Mode	41

9	Braille	42

9.1	Getting Started	42

9.2	Entering and Editing Text Using Braille	42

10	Magnification	43

10.1	Exploring the Mobile Phone Screen	43

10.2	Starting and Stopping Mobile Magnifier	43

10.3	Configuring Mobile Magnifier	43

10.4	Magnification Lens (Touch screen devices only)	43

11	User Dictionary	44

12	Browsing the Web	45

13	Third-Party Application Support	46

13.1	QuickWord	46

13.2	Fring	47

14	Ovi Maps Support	48

14.1	Supported Devices	48

14.2	Basic Operation of Ovi Maps v3.03 and v3.04:	48

14.3	Shortcuts when in Map, Walk or Drive Mode:	49

14.4	Maps Configuration Tab in Configure Mobile Speak:	50

14.5	Known Issues with Ovi Maps v3.03 and v3.04:	50

15	EULA	51

16	Contacting Code Factory	55

17	About Code Factory	55

Appendix A: Complete Command Reference	56

18	Keyboard Commands	56

18.1	Default Layout	56

18.2	Reading Layout	56

18.3	Magnification Layout	57

19	Touch Commands	58

19.1	General Layout (Accessed from Quadrant Mode)	58

19.2	Go To Layout (Accessed from Quadrant Mode)	58

19.3	Settings Layout (Accessed from Quadrant Mode)	58

19.4	Speak Layout (Accessed from Quadrant Mode)	58

19.5	Review Cursor Mode	59

19.6	Command List Mode (Accessed from Quadrant Mode)	59

19.7	Virtual Touch Keyboard (Accessed from Quadrant Mode)	59

19.8	Stylus Mode	59

Appendix B: Braille Commands	60

20	BAUM Conny / VarioConnect / Pronto	60

21	BAUM SuperVario / HumanWare Brailliant	62

22	HumanWare BrailleConnect Series	64

23	Optelec EasyLink and EL12	66

24	Optelec ALVA Braille Controller 640	68

25	Eurobraille Esys	70

26	Handy Tech Braillino and BrailleStar	72

27	Handy Tech BrailleWave	74

28	Handy Tech EasyBraille	76

29	Freedom Scientific Focus Blue	77

# Introduction to Mobile Speak

Thank you for choosing Code Factory products to provide you with the most comprehensive access to mainstream mobile technology!

Mobile Speak is the first screen reader in the world to support both the Windows Mobile and Symbian platforms. It allows you to access all of the functionality of your mobile phone, including:

- Making and receiving calls

- Composing and reading text messages (SMS) and e-mail

- Browsing the Internet

- Listening to music and other audio content

- Scheduling  appointments and tasks

- Managing your contacts and call lists

- Checking the battery level, signal strength, Bluetooth state, and other status details

- Configuring phone settings, wallpapers, profiles and ringtones

- Accessing a wide range of third-party applications

This documentation aims to provide a basic understanding of how to use Mobile Speak to access a Symbian S60-based smartphone. It should be considered as an overview of the available functions and features of the software, not of the devices on which it runs. For more information about your mobile phone, please consult the user manual or contact your mobile operator’s customer service department.

For additional information on Mobile Speak and all Code Factory products, you can search the Code Factory Knowledgebase. This Knowledgebase contains answers to frequently-asked questions regarding how to perform certain tasks with the help of Mobile Speak.

You may also wish to subscribe to one of the Mobile Speak mailing lists. The Code Factory mailing lists serve as a rich source of information, as other users of our products are there to help answer questions, share tips and tricks they have learned from personal experience or other resources, and suggest ways to troubleshoot issues you may be encountering. These mailing lists are also monitored by Code Factory managers and developers for feature requests, reported bugs, suggestions and comments on how to improve the product, and other forms of constructive feedback. For more information and to subscribe, please visit http://www.codefactory.es/en/list.asp?id=88.

# What’s New in 4.0

Mobile Speak 4 is cross-platform, touch, multi-voice, and ready for the next generation.

## Cross Platform

With Mobile Speak 4, you can switch phones as many times as you want, for free. In addition, Mobile Speak and Mobile Magnifier are not only cross-device, they are also cross-platform, which means you can transfer your Mobile Speak or Mobile Magnifier license from a Symbian to a Windows Mobile device and vice versa.

In Mobile Speak 4, your mobile phone number is your user ID. Therefore, as long as you do not change phone numbers you can transfer your Mobile Speak or Mobile Magnifier license between devices, free of charge, as many times as you wish. There is also no need to deactivate your license from the old device before activating it on a new device.  Just enter your phone number on the new device and your license will automatically be activated. Likewise, if your phone is stolen, broken or lost, as long as you do not change phone numbers, you do not have to send Code Factory any kind of certificate - just install the software on your new device and activate it with your User ID.

## Touch

With Mobile Speak 4, you have three different ways to interact with your touch screen mobile phone:

- Use the classic keyboard commands.

- Use a brand new system of easy touch screen commands

- Move your finger around the touch screen and let Mobile Speak tell you what is below your fingertips.

The Mobile Speak touch interface has been completely redesigned - it is now much more powerful and intuitive than Mobile Speak Pocket, its predecessor. It also works on Symbian as well as Windows Mobile, so it is compatible with phones such as the Nokia N97, X6, C6, Nokia 5800, Nokia 5230, and Nokia 5530.

On phones with hardware keys as well as a touch screen, you decide what is most convenient for you, using touch screen gestures or pressing keys.

Mobile Speak 4 also allows you to activate a review cursor mode, similar to VoiceOver for the iPhone. When you move your finger around the screen, Mobile Speak will speak what is under your finger and you only have to double tap to activate the item.

Whether you choose the quick access of touch screen gestures, the easy navigation of the review cursor, or the traditional keyboard shortcuts, or a combination of all three, the choice is yours. You can change the method depending on which mobile device you are using, your preferences, or even specific situations.

## Multi-Voice

When you buy Mobile Speak 4 license, you get three voices for the price of one.

With Acapela and Loquendo, Mobile Speak 4 allows you to use up to three voices from a single Text-To-Speech provider. That means that you can use either three Acapela or three Loquendo voices with only one Mobile Speak license. Further, no matter which voices you choose to install first, you are not tied to them. As long as you do not use more than three voices at the same time, you can switch them as many times as you like. However remember that this feature is tied to the Text-To-Speech provider, so you will have to choose if you prefer to have three voices from either Acapela or Loquendo because you will not be able to mix voices from both providers.

## Ready for the Next Generation

With Mobile Speak 4, you do not have to worry about being left behind with old technology. Mobile Speak supports the latest mobile phones such as the Nokia N97 and HTC Touch Pro 2, as well as the latest versions of Windows Mobile, V6.5 and of Symbian, S60 5th Edition. To check the full list of supported devices, and to know which devices are considered as the most popular among Mobile Speak users, please visit our Web site at http://www.codefactory.es.

## Packed with New Features

In addition to supporting the latest handsets, Mobile Speak 4 also offers many new features, including:

- New keyboard shortcuts.

- The possibility to add new user profiles and assign a specific voice to a specific application (calendar, messages, contacts, etc).

- Back light option - disable or reduce the backlight to save battery power.

- Stop Mobile Speak while it is speaking.

- Support for 8 of the latest Acapela voices: Dutch Belgian Jeroen, French Margaux, Greek Dimitris, Norwegian Olav, Portuguese Brazilian Marcia, Russian Alyona, Spanish Antonio, and Turkish Ipek.

The Symbian version of Mobile Speak 4 includes new features such as:

- Advanced support for text editors such as Quickword, LightNotepad, and Nokia built-in Notepad.

- You can now edit a document as well as read by line, sentence, words, and characters.

- Advanced support for Fring, the free messaging application. Mobile Speak allows you to use Fring to make VOIP calls as well as chat with friends.

- Support for Vocale Presse, an accessible application to read newspapers.

- Configurable volume limits.

- Phonetic spelling mode.

- Improved built-in user dictionary: ability to add a word as case dependant (useful for abbreviations for example), or words with punctuation symbol.

- Support for new Nokia devices: 5730, 6700 Slide, 6710, 6720, 6730, 6760, 6790 Slide, 6790 Surge, E52, E72, N86, 5230, 5530, 5800, N97, N97 Mini, C6 and X6.

- Supports for new Arabic Youssef Acapela voice.

- Support for Galop, a new Czech TTS provider.

## New features, improvements and fixes in v4.60

The latest v4.60 of Mobile Speak introduces many new features, improvements and fixes. These include:

- Support for Nokia Ovi Maps v3.03 and v3.04. These versions provide users with free, voice-guided turn-by-turn assistance in both pedestrian (Walk) and car (Drive) modes.

- Support for Mobile Magnifier in various 3rd edition phones where it previously was not supported: N86, E75, N79, N85, 5730, 6220.

- Support for Mobile Magnifier in all Touch devices. As well as providing all of the Magnifier features offered on 3rd edition devices, on 5th edition devices a new Touch Magnification Lens is provided.

- Touch phones: The setting for "Touch mode on startup" has been changed to "Touch mode", so that it reflects the current Touch mode. Pressing Command + Volume up/down will change this setting now in the current Mobile Speak profile. When changing Mobile Speak profiles, the touch mode will change to the mode configured in the newly active profile.

- Touch phones: On pressing the Command key, the touch mode will automatically change to Keypad mode while the Command key is active, regardless of the current touch mode. This allows all Command key shortcuts to always be easily accessible, since all keys can be simulated in Keypad mode.

- Touch phones: On phones that have a Media Key (key to the upper right of the touch screen) like the 5230, 5530, 5800 and X6, it is now disabled by default if not in Stylus mode, to prevent it being pressed by accident.

- Touch phones: When typing in Keypad mode, consecutive taps that are close to each other but that occur on 2 different virtual keys will now always be treated as multiple taps of the first key. This avoids problems when typing and tapping near the border of virtual keys.

- When "Batt/Signal always" is set to "On", the current time, battery level, signal level and operator will be spoken when entering into the standby screen from a different application. This is a convenient way to check these values without needing to use a Mobile Speak Command (so it can easily be done for example by switching out and back into the standby screen, even on slider phones like the N86, while keeping the slider closed).

- Support for new phones: Nokia E55, 5228, 5250

- Web browser: pressing Joystick Enter when at the start of a list or table will now jump to the end of the table, and vice versa.

- Improved general stability of Mobile Speak and Mobile Magnifier.

- Fixed problem on some phones where Mobile Speak was sometimes saying "Unlock" after a key was pressed with phone locked, even with "Mute on keypad lock" set to On.

- In T9 Predictive mode: When pressing the 1 key to enter a '.' it will be spoken now, regardless of the current punctuation setting.

- In T9 Predictive mode: When pressing the * key after having pressed the 1 key to scroll through the list of punctuation characters, they will be spoken now, regardless of the current punctuation setting.

- Fixed problem when "Auto case change" was set to Off, which prevented the input status from being spoken when pressing # inside an editor on Touch phones, and also in some other situations where it should have been spoken.

- Fixed bug where editor's input status was sometimes not being spoken.

- TweetS60: Fixed problem in new version 1.48.2 where sender was sometimes not being spoken correctly.

- Fixed problem in non-touch phones with dialpads where pressing the * key more than once from the dialing screen was not cycling through the *, +, p and w keys.

- X6: Fixed problem that prevented email list from being spoken.

- Touch phones: Reading list of songs in Music Player now.

- Touch phones: Reading "To" label now when writing SMS messages.

- Touch phones: Fixed problem in phones without QWERTY keyboards, where it was not possible to dial the * key from the dialing screen.

- Touch phones: Fixed problem where pressing Command + Volume up/down was not working from some applications (e.g. dialing screen).

- Touch phones: Fixed problem where pressing Command + Volume up/down and then another Command key before the first one had timed out was treating the second press of a command key as a double press of the key and performing its original function.

- Fixed problem with mini-symbol table on some new firmware versions (e.g. E52 with v51.x firmware).

- Fixed stability issues which could occur in some views in Configure Mobile Speak.

- Fixed problem where the battery and signal level was sometimes spoken unexpectedly from the standby screen.

- Fixed problem where list item's "marked" status was being spoken incorrectly inside Settings > Phone > Application Settings > Positioning > Positioning methods.

- Keyboard help mode: fixed problem where Volume up/down key presses and (on supported devices) multimedia keypresses were not being detected in keyboard help mode when launching keyboard help from certain applications (e.g. Standby screen).

- Fixed problem which could cause some applications to close unexpectedly (e.g. Web browser on some devices in Greek, Chinese).

- Fixed problem in some firmware versions of the E72 (e.g. some German and French firmware) where some numbers were not able to be dialed correctly from the dialing screen.

- Braille: Changed some commands on Alva devices to make use of the device easier.

- Braille: Fixed problem on some devices where key commands needed for some Mobile Speak Commands were getting switched.

- Braille: Fixed problem with the Enter key on Handy Tech Easy Braille device.

- Braille: Fixed bugs when sending commands from Alva.

# Installation on Symbian Devices

If you have obtained your mobile phone from a distributor, Mobile Speak / Mobile Magnifier will already be installed on your device. However, it is possible that you may have had to remove the software from your system, or that you have downloaded the software from the Internet and want to install it yourself.

## Prerequisites

Prior to installation, ensure the following:

- The date and time must be correctly set on your phone. Otherwise, the packages may not install successfully and the trial license may not function properly.

- That there is sufficient free memory available on the device (either in the phone memory, memory card, or the hard disk). The Mobile Speak package requires around 8Mb of free memory, while the voice package sizes can vary (Acapela HQ voices are the largest and may occupy up to 25 Mb).

- That the firmware on your phone is up to date. To check your current firmware, please dial *#0000#. The earlier firmware versions on many Symbian phones have issues that interfere with the operation of your phone, with and without Mobile Speak installed. To update your firmware, either go to your Nokia service center, or (if your phone is on the list of supported phones) do it online using the Nokia firmware update service at http://www.nokia.com/A4176089. To check what firmware is available for your phone via the Nokia firmware update service, first go to http://europe.nokia.com/A4305060 and enter your phone’s product code (a seven-digit code found inside the phone, on a label underneath the battery). Most S60 phones are supported by this service.

- That your phone is using the correct security setting. Note that on some phones (especially E-series or operator-branded models), an additional security setting exists. The default setting may need to be changed before installation of any application is permitted on the phone. To do this, go into the phone’s Application Manager and press Options / Settings and make sure that Software Installation is set to “ON”, and Online Certificate Check is set to “OFF”.

## Installation

The following instructions apply to phones where Mobile Speak has never been previously installed, or where all Mobile Speak packages have been removed. If you are upgrading from a previous version of Mobile Speak, please read the section on upgrading below before returning to this section to complete the installation.

All installation packages can be found on the Code Factory web site. The base Mobile Speak package has a name such as “MobileSpeak_S60_v4.6.SIS”. It contains all the files needed to run Mobile Speak with the Nokia voices, and also to run Mobile Magnifier. To use other voices, you can install TTS packages which can all be found on the Code Factory web site. In general, the TTSs have a single installation package (e.g. “english_us-heather_22k_3rd.SIS”). The Loquendo voices are an exception – they consist of two packages - loquendo_common_3rd.SIS and a Loquendo voice file. Please install the loquendo_common_3rd.SIS file first, and then one or more Loquendo voice files.

The order of installation is not important, so you can install the Mobile Speak package prior to installing the TTS or vice versa. You may install the packages to the phone memory, memory card, or (where available) hard disk of the phone, as long as sufficient free space is available.

There are several ways to transfer these files to the phone and install them. The choice of which method to use depends on the user. Keep in mind that some of these techniques may require sighted assistance, as several questions will be asked on the phone during the installation process. The possible choices include:

- PC Suite: this is a free PC software package from Nokia, which comes with the phone or can be downloaded free of charge. The phone is connected to the PC using a USB cable, and you must connect the phone in “PC Suite” mode after connecting the USB cable. Please refer to the PC Suite reference manual for a full description of this software.

- Bluetooth: a Bluetooth pairing must first be set up between the phone and the computer. Once paired, the installation packages can be transferred to the phone. They will be sent to the Inbox and the installation process will begin when the message is opened. Remember to delete each package from the Inbox after installation.

- Web: by navigating on the phone’s Web browser to the Code Factory web site, the installation packages can be downloaded directly to the phone. Installation will be started automatically once the download is complete.

- File transfer: you can also transfer the installation packages directly to the phone (e.g. by copying the files to the memory card using a card reader). Once this is done, use the phone’s File Manager to locate the installation package. By opening it, the installation process will start automatically. Remember to delete each package after installation.

After a successful installation, Mobile Speak will start automatically on most models. However, it is advisable that you restart the phone when installation is done. If Mobile Speak did not start automatically, please use the “Start Mobile Speak” shortcut to launch the application.

Mobile Speak comes integrated with Mobile Magnifier. By default, Mobile Speak is activated and Mobile Magnifier is deactivated the first time that the Mobile Speak package is installed. However, you can then customize the startup behavior of Mobile Speak and Mobile Magnifier by configuring the Auto Start Mobile Speak and Auto Start Magnifier options in the Configure Mobile Speak application.

## Uninstalling

To uninstall Code Factory products, use the Manager application, which in most phones is located in the Settings or Applications folder. The shortcut to this program is sometimes labeled “App Mgr”, “App Manager”, or “Application Manager”.

The Manager application will show a list with all the packages and applications installed in the system. Remove the Mobile Speak package first, and then any voices, games or tools. It is not necessary to first press “Stop Mobile Speak” before uninstalling.

After uninstalling the packages, Mobile Speak will have been cleared from your mobile phone. It is recommended that the phone be restarted after uninstalling Mobile Speak.

Note that on Symbian 3rd edition phones, it is also possible to uninstall Mobile Speak by pressing the Clear key on any of the Mobile Speak icons.

## Nokia Built-In TTS

Mobile Speak provides access to the Nokia built-in TTS on all Nokia Symbian 9 phones. The standard voice is called "Nokia" and, on phones that support the Nokia High Quality TTS (i.e. Nokia E50, E51, E52, E55 E61i, E63, E65, E66, E71, E71x, E73, E72, E75, E90 Communicator, N75, N78, N79, N81, N85, N86, N96, N97, 5320, 5500 Sport, 5530, 5630, 5700 XpressMusic, 5730, 6110 Navigator, 6120 classic, 6121, 6124, 6210, 6220, 6290, 6700 Slide, 6710, 6720, 6790, C5, C6, E5 and X6 phones), another called "Nokia HQ" is also available. These Nokia voices are shipped with all Symbian 9 phones, and new Nokia HQ voices are also downloadable on supported phones from the Nokia web site at http://europe.nokia.com/A4286225. The Standard Quality TTS will automatically switch languages to match the phone language. The High Quality TTS can be configured to use a specific language and voice or modify the speed via the "Speech" application, which is usually located inside the phone’s Settings folder.

If you have a supported phone, you can download and install additional High Quality voices. To do this, visit the Nokia download page at http://europe.nokia.com/A4286225. For a given language, you must install the language package and at least one of the voices listed. However, most phones will come with the TTS of the default language already installed. After installing the new voice, you’ll see it listed inside the phone’s Speech application. Use the Speech application to manage the currently selected voice. The volume and speed will be controlled by Mobile Speak.

A FAQ regarding the Nokia built-in TTS can be accessed at http://www.codefactory.es/additional_docs/en/nokiaTTS_EN.html.

## Software Updates

Updates to the Mobile Speak software are regularly made available at the Code Factory web site www.codefactory.es. Minor updates are usually free, while major updates may require purchase of an upgrade license, so check the release notes for each version to see whether this is required.

## Upgrading from a Previous Version

Users who are upgrading to v4.xx from v3.xx MUST first uninstall ALL Code Factory packages (TTS, Mobile Speak and any related tools) before installing v4 of Mobile Speak. This is extremely important, since failing to do so may prevent the products from working correctly, or may even cause phone instability. Check with your distributor, or refer to the “Uninstalling” section above, if you have any questions regarding how to remove packages from your phone.

Once all Code Factory packages have been uninstalled, restart the phone, and then you can proceed with a standard installation, as described in the “Installation” section above.

Users who are upgrading from v4.X to the current version should first stop Mobile Speak. If installing to the same memory location, then it should be possible to install the current version without uninstalling the previous version. However, if you wish to install to a different memory location, then it is recommended to first uninstall the previous version. Note that none of the TTS packages have changed since v4.0, so it will not be necessary to reinstall the TTS packages.

# How-to's

On touch screen devices, keep in mind that all Command key shortcuts that are described in this section can be performed using the virtual keypad when in Keypad mode.

## How Do I Make a Phone Call?

To make a phone call, you should first navigate to the Standby screen.  You can switch between the Standby screen and the Main Menu by using the “Menu” key.

All numeric keys that are pressed in the Standby screen are interpreted as digits for the telephone number to be called. When all the digits of the phone number have been entered, press the Send key, or open the Options menu with the left soft key and select “Voice Call” to dial the entered phone number.

On touch screen devices with no physical keyboard, the most convenient way to dial a phone number is to enter into Keypad mode and dial the number directly from the standby screen, then press the Send key. Alternatively, you can use the virtual keyboard in number or QWERTY modes to enter numbers directly and make a phone call.

To answer an incoming call on a touch screen device (regardless of lock screen status): while the phone is ringing, slide left to hear the caller ID, and slide right to answer call directly. Note that on some phones, the slide left with phone lock may not read the caller ID - in this case it will be necessary to unlock manually using the phone's keyguard switch key.

## How Do I Access the Main Menu?

You can switch between the “Standby” and “Main Menu” windows by using the “Menu Key”. From the “Main Menu”, you can access all the applications on the phone, which are located either in the current “Main Menu” window itself or in subfolders.

By long pressing the “Menu Key”, you can display a list of currently running applications. You can browse this list by going up and down with the joystick key. With the center joystick key or with the left soft key, you can select the currently selected application. With the right soft key (“Cancel”), the list will be closed without selecting anything.

## How Do I Send an SMS?

You can switch between the “Standby” and “Main Menu” windows by using the “Menu Key”.

Sending an SMS from the “Main Menu” Window

In the “Main Menu” window, you can navigate the various Menu folders with the joystick to locate the “Messaging” icon. Press the joystick to open the “Messaging” application.

In the application window, you can browse the menu. The first (uppermost) menu option is “New Message”. Select this option to choose among “Text Message” (the default), “Multimedia message”, “Audio message”, and “E-mail.” Selecting “Text Message” will display the “Text Message” window.

When the “Text Message” window opens, the cursor initially is located in the “To” field. In this field, the default input mode is number mode, as normally you would want to enter the telephone number of the recipient. If you press the joystick in this field, the list of “Contacts” appears where you can scroll through this list to choose the recipient.

Press downwards on the joystick to place the cursor in the input text field. Here, you can write your text message. Press upwards on the joystick at any moment to return to the “To” field.

With the left soft key, you can access to a list of options which you can navigate and select with the joystick. The most important options are:

- “Send” - this only appears when the “To” field is filled. Selecting Send will send the text message to the recipient(s) listed in the “To” field.

- “Add recipient” - opens a list of Contacts, from which the recipient can be selected.

- “Exit” - exits the entire “Messaging” application.

Note that the Messaging application may also be assigned to either the left or right soft key, or as an active shortcut, in the phone’s standby/home screen.

## How Do I Manage My Contact List?

You can manage your list of email and phone contacts via the Contacts application. In both cases, the Contacts application can be found in the main phone menu, and may also be assigned to either the left or right soft key in the phone’s standby/home screen.

Once you have opened the Contacts application, you will be presented with a list of your current contacts. By selecting a contact, you can choose to make a voice call, send a message, or send an email, provided that you have stored all of the relevant information for each of these options. Depending on the operating system and the capabilities of the device, you may also have the option to make a video call, send the contact information via Bluetooth, record a voice dialing prompt, or any number of additional operations.

## How Do I Manage Calendar Appointments?

The Calendar application can be found on Symbian phones in either the “Main Menu” window or in the “Applications” subfolder.

The Calendar application opens with the “Month View” window, with the cursor on the current day. This current date is spoken at startup. With the joystick key, you can scroll through the days of the month and move to the previous or the next month.

You can change between “Month View”, “Week View“, “Day View”, and (on supported devices) “Agenda” and To-do” views using the Options menu.

Creating a New Appointment

There are three ways to create a new appointment:

- Press the left soft key to open the Options list and then select the “New Entry” option.

- Press the left soft key to open the Options list and then select the “Open” option. This will open the “Day View” where you can open the “Options” list and select the “New Entry” option.

- Press the joystick to open the “Day View”. Here you can open the “Options” list and select the “New Entry” option.

Once the “New Entry” option is selected, you can choose between creating a “Meeting”, “Memo”, “Anniversary”, or a “To-do” type of event.

In the “New Appointment” window, there are various fields available depending on the type of appointment event. The label of the highlighted field is spoken, and you can select the current field by scrolling through the window with the joystick key.

Pressing the right soft key (“Done”) saves the new entry and you are returned to the “Day View”.

Editing an Appointment

In the “Month View” and “Week View” windows, the user can browse the calendar days using the joystick key. When a day is selected containing one or more appointments, the voice - after mentioning the date itself - will speak the word “Meeting”.

By selecting “Open” from the Options menu or double-pressing the joystick, “Day View” is displayed where a list of all the appointments (meetings, memos, to-do’s and anniversaries) is shown. When scrolling through this list, the subject, location, and starting time of the highlighted item will be spoken.

When selecting “Open” in the Options menu in “Day View”, the currently highlighted appointment event will be opened. As when this item was created, the various fields (depending on the type of event) can be scrolled through and edited with the joystick.

Again, with the right soft key (“Done”), the newly edited entry will be saved and you will return to “Day View”.

Deleting an Appointment

By selecting Delete from the Options menu in “Day View”, the highlighted item in the appointments list will be deleted. Before deleting it, you will be prompted to confirm.

## How Do I Read Text Content?

There are various shortcut commands you can use to read text content. These commands help you navigate the text, by repeating certain parts of the text or spelling certain words. For most of these commands, Mobile Speak must be in the “Reading Layout”. First press the Mobile Speak Command key, followed by the specified key.

ALL LAYOUTS

Key 		Short press command 		Long press command

0 			Mute on/off

5 			Change command layout

DEFAULT LAYOUT

Key 		Short press command 		Long press command

2 			Read current focus

8 			Spell last thing

# 			Read screen

READING LAYOUT

Key 		Short press command 		Long press command

1 			Previous sentence

2 			Read from current focus 		Move to start of text

3 			Next sentence

4 			Previous word

6 			Next word 			 Current word (Symbian)

7 			Previous paragraph

8 			Read from current position 	Move to end

9 			Next paragraph

# 			Read screen 			Spell current word (Symbian)

## How Do I Check the Battery and Signal Levels?

Use Command + left soft key or, on touch screen devices when in Quadrant mode, Tap 4 (Speak) and then double Tap 2 to hear the battery and signal strength.

## How Do I Lock and Unlock the Phone?

You can switch between the “Standby” and “Main Menu” windows by using the “Menu Key”.

When in the “Standby” window, on most non-touch devices you can lock the keypad of the phone by pressing the left soft key and subsequently the asterisk key (*). To unlock the keypad, repeat the same steps. Note that on some devices, the key combination may be different, e.g. left soft key + Function key; left soft key + right soft key. An alternative way to lock the phone is to press the Power key, scroll down 1 item to “Lock keypad” and press joystick enter. Note that some phones may have a special sliding key (generally located on the side or on the top of the phone) that will lock/unlock the keypad and/or touch display. This is available on all touch-screen devices and is called the Keyguard switch.

## How Do I Access the Mobile Speak Control Panel?

The Mobile Speak Control Panel (called “Configure Mobile Speak” in Symbian) can be accessed at any time by using any of the following:

- Pressing the Command key + right soft key

- Tap 2 (Settings) and double tap 3 on touch screen devices when in Quadrant mode.

Additionally, it can be accessed by navigating to the “Code Factory” folder in the main menu, and opening the Configure Mobile Speak application.

## How Do I Change the Stylus Mode? (Touch screen devices)

Activating Stylus mode allows you to use the touch screen normally, bypassing all of the Mobile Speak touch commands. This may be useful when the device will be used by a sighted person who might otherwise have difficulty navigating using the touch commands.

To activate Stylus mode, repeatedly press Command key + Volume Up or Command key + Volume Down to move through the list of touch modes until the Stylus mode is enabled.

To return to the original touch mode, repeat the same process until the desired mode is enabled. Alternatively, the current MS profile’s “Touch mode” setting can be set to the desired mode.

## How Do I Mute Mobile Speak?

To mute Mobile Speak, press Command Key + 0 or, on touch screen devices when in Quadrant mode, use Tap 2 (Settings Layout) and then Tap 1.

To resume speech or un-mute, use Command Key + 0 or, on touch screen devices when in Quadrant mode, triple tap.

## How Do I Stop Mobile Speak?

To stop Mobile Speak:

- Navigate to the “Main Menu”.

- Navigate to the “Code Factory” folder in the Main Menu using the joystick.

- Press the center joystick to open the “Code Factory” folder.

In the “Code Factory” folder, there are various options which may include:

- Stop Mobile Speak

- Configure Mobile Speak

- Stop Mobile Magnifier

- Start Mobile Magnifier

- Start Mobile Speak

- Select “Stop Mobile Speak” with the center joystick.

To restart Mobile Speak, navigate to “Start Mobile Speak” and press the center joystick.

# Licensing

Mobile Speak 4 comes with a new licensing system which offers many benefits and allows maximum flexibility. Users can choose between two licensing methods, one based on your telephone number (known as phone-number-based licensing), and one based upon a unique identifier for your device (known as device-based licensing).

A detailed description of each of these licensing methods is available on our web page at http://www.codefactory.es/en/products.asp?id=309.

## Activation Manager

After installing Mobile Speak or Mobile Magnifier, you can activate the products or obtain a trial license through the Activation Manager. The Activation Manager is launched when entering Configure Mobile Speak > Options > Product Activation.

Before attempting to activate the product, please make sure that your phone has service and is not in offline or flight mode, and that the date and time are set correctly.

The Activation Manager will communicate with the Code Factory licensing server using your preferred method of connection:

- Via the Internet using your device's connection with a Wi-Fi hotspot.

- Via the Internet using your mobile phone's Internet service. Please consult your mobile phone provider about configurations on how to get this to work, as well as any charges for using the service.

- Via SMS/text message. Please consult your mobile phone provider regarding charges for sending and receiving SMS from international numbers. Refer also to our list of mobile phone networks confirmed to work with SMS activations.

## Getting a Trial License

Getting a trial license is very straightforward in Mobile Speak, and does not require any help from your distributor.

- Enter the Activation Manager.

Upon entering the Activation Manager on a Symbian device, you will be asked to select your preferred method of connection (SMS or Internet). After entering the Activation Manager, some informational prompts will be displayed, warning of possible SMS or Internet charges involved in the activation process.

- Select “No” when asked if you are a registered user of Code Factory products.

An activation request for a trial license will be sent to the Code Factory server. The request will be done for all Code Factory products installed on the device at the time of activation. Depending on the connection speed, this process may take some minutes. The activation process will then complete and an appropriate information message will be displayed. After completing the activation, the license status for all products can be found in Configure Mobile Speak > Options > About.

If additional voices or Code Factory products are installed on the device at a later time and you wish to obtain a trial license for them, it may be necessary to repeat the trial activation process in order to obtain the trial license.  Check the license status first if you are unsure.

Trial licenses are time and date sensitive. Therefore, changing the time or date on your phone may cause a trial license to expire. If this occurs, simply set the correct time and date and repeat the trial activation process and any remaining days of the trial will be reactivated.

## Activating a Full License

The process for activating a trial license is very similar to that of obtaining a trial license. After purchasing a full license for your product, enter the Activation Manager, select your preferred method of connection and respond to any informational prompts. Answer “Yes” when asked if you are a registered user of Code Factory products. If connecting via the internet, you will be prompted to enter your phone number if you are registering a product with a phone-number-based license (please leave this field blank if you have a device-based license). Press “OK” (or “Yes”) to send the activation request to the Code Factory server. Once completed, check the license status of all Code Factory products to confirm that the activation has been successful.

When activating a full license, a trial license will also be generated for any other installed products that had not previously been trialed and for which you have not yet purchased a license.

If at a later time you decide to install additional Code Factory products, or you re-install existing products, you may be required to repeat the activation process.

# Commands

Mobile Speak allows rapid access to phone information as well as configuration options via the Mobile Speak Command Key. Pressing the Command key followed by an appropriate key will perform a specific command (e.g. Command + Left soft key to read phone status information). Mobile Speak Commands are available on all 3rd edition phones. They are also available on 5th edition touch phones, either by using the built-in keyboard or when using the device in Keypad mode.

The default Command key changes depending on the phone model. Please refer to the “Command Key submenu” section in Chapter 7 for this information. It can be configured in Configure Mobile Speak > Options > Command key.

Note that in all Touch phones, the touch mode will automatically change to Keypad mode while the Command key is active, regardless of the current touch mode. This allows all Command key shortcuts to always be easily accessible, since all keys can be simulated in Keypad mode.

For a full list of the Mobile Speak commands available by pressing the Mobile Speak Command key, please refer to the Keyboard Commands chapter in Appendix A.

## Keyboard Help Mode

The Keyboard Help Mode familiarizes you with the keyboard layout as well as with Mobile Speak’s commands. In this mode, you can explore all the commands without actually performing them.

To turn on Keyboard Help Mode, press the Command Key and then press and hold the 5 key for 1 second. Mobile Speak will say "Help Mode On". You can now press any key to learn about the keyboard layout or perform a touch gesture. To learn about the Mobile Speak commands, press Command and then a short or long press of a key. Mobile Speak will provide a short description of the command assigned to that keystroke. To turn off Keyboard Help Mode, press the Command Key and hold the 5 key for 1 second again. Mobile Speak will say "Help Mode Off". Help mode will also turn off automatically if no input is received during 15 seconds.

On touch phones, note that when in Keypad mode, the Keyboard Help mode provides an excellent way to become familiar with the virtual keys and slide gestures available in these modes, as well as the Mobile Speak commands. It is highly recommended to new users of Touch devices to spend some time in Keyboard Help mode, mastering the location of the keys and the correct way to perform gestures.

## About Layouts

Mobile Speak 4 organizes commands into groups called “layouts”. These groups combine related commands, such as those dealing with magnification or settings, in order to make them easily accessible and make optimal use of the limited number of keys available on some mobile devices.

The following three layouts are available on both touch screen and non touch screen devices:

Default	Includes the most commonly-used commands.

Advanced Reading	Provides commands for text navigation, allowing you to navigate by character, word, line, paragraph, etc.

Magnification	Includes the most commonly-used commands for controlling Mobile Magnifier. Note that this layout is only available on phones that support Mobile Magnifier, and can only be entered into when Mobile Magnifier is active.

The Default layout is active by default when Mobile Speak is started. By pressing Command + 5, the active layout will be changed to the next one. For a list of all the Keyboard Commands available in these layouts, please refer to the Keyboard Commands section in Appendix A.

In addition, on touch screen devices, Mobile Speak uses the following touch layouts when in Quadrant mode:

General	Includes the most commonly-used commands, such as moving the cursor, as well as switching between other layouts.

Go To	Allows you to navigate to various parts of the phone interface, such as the left and right soft keys, the main phone menu, Configure Mobile Speak, and so on.

Speak	Offers commands to speak information about the phone and the current application, as well as spelling and repeating spoken text.

Settings	Allows you to change the most frequently-used Mobile Speak settings, such as volume and speed of speech output, without going into Configure Mobile Speak.

The General layout is active when Quadrant mode is entered. By performing the appropriate tap, one of the other layouts will be entered into. For a list of all the Touch Commands available in these layouts, please refer to the Touch Commands section in Appendix A.

# Configure Mobile Speak

Mobile Speak and Mobile Magnifier are unique because of the flexibility they offer you in customizing your mobile access solution. With more than 60 user-configurable options for speech, Braille and magnification, you can have an accessible phone best suited to your needs and preferences.

## Mobile Speak Configuration

To configure Mobile Speak and/or Mobile Magnifier, navigate to and enter the “Code Factory” folder.

In the “Code Factory” folder, there are various Mobile Speak options which may include:

- Stop Mobile Speak

- Configure Mobile Speak

- Stop Mobile Magnifier

- Start Mobile Magnifier

- Start Mobile Speak

Select “Configure Mobile Speak” and enter to open the application.

## MS Profiles

Mobile Speak organizes configuration settings into groups called MS Profiles, allowing you to switch between different profiles as you need them. For example, you could configure one MS Profile to use a certain voice, speed, and level of verbosity for reading long passages of text, instead of your preferred configuration for normal use of the phone. All of the Mobile Speak settings are included in each MS Profile, making it easy to customize them to your specific needs in different situations.

In the initial screen of the Mobile Speak configuration panel, you will see the three MS Profiles, named by default as Normal, Beginner, and Advanced. Additionally, you can define your own personal MS Profiles.  One of these MS Profiles is always marked as Active, which means that its settings are currently in use. The word “Active” will be spoken after the name of the currently active MS Profile.

For each MS Profile, you have the option to activate or customize it. By selecting the Customize option, you will be presented with various tabs, each of which contains a different set of configuration options, as discussed in the next sections of this chapter.

## Configuration Structure

Each MS Profile is divided into 6 groups of settings: Speech, Reading, General, Verbosity, Braille, and Magnifier (on supported phones). To switch between the different groups, use the joystick left and right keys, or slide left and right if using a touch screen device when in Quadrant or Keypad mode. The options that also have associated “Command key” shortcuts are described in the “Overview of Command Key shortcuts” section.

Note that any changes that you make to the configuration settings will only take effect once you exit the MS Profile that you are currently customizing or when you switch to a different setting group using the joystick left or right key or left/right slide gestures.

Important: Do not change settings (e.g. Volume, Verbosity) using “Command Key” shortcuts while the Configure Mobile Speak application is running. These changes will be overridden when you exit the Configure Mobile Speak application.

## Speech Settings

The following is a list of the Speech configuration options:

- Change the voice, if you have installed several voices.

- Change Volume from 1 to 10.

- Change Speed from -10 to 10.

- Change Spelling Speed from -10 to 10.

- Change Pitch from -10 to 10.

- Change the Voice on call.

- Change the Volume on call.

- Toggle the “Silent on calls” option.

- Toggle the “Mute on Keypad lock” option.

- Toggle the “Silent while dialing” option.

- Change the Audio Profile.

- Change the Speech delay (seconds).

- Reduce minimum volume.

- Increase maximum volume.

Voice

Many different voices can be installed on the same phone, and the user is able to switch between them. All installed voices appear in the selection list, however if you choose one that is not activated, it will expire after 10 minutes. In the case of Loquendo and Acapela, if you install more than three voices, only three will be shown in the list, and you have to uninstall one of these three before you can use a different one. By selecting the voice “none”, no speech will be heard when this profile is active.

Volume

This option increases or decreases the volume of the speech output (1 to 10). It does not affect the volume of system sounds or the audio during a call. It also has a related shortcut “Command Key” combination (“Command Key” + joystick key up, “Command Key” + joystick key down, in default and reading layouts), as well as touch screen gestures to change the volume. See the Section with the overview of Command Shortcuts for a further description. The default level is 5 for all MS Profiles.

Speed

This option increases or decreases the speed of the speech output (-10 to 10). It also has a related shortcut “Command Key” combination (“Command Key” + joystick key left, “Command Key” + joystick key right, in default and reading layouts), as well as touch screen gestures to change the speed. See the Section with the overview of Command Shortcuts for a further description. Default is 0 for the Normal MS Profile, 0 for the Beginner MS Profile, and 5 for the Advanced MS Profile. Note that the speed of the Nokia HQ voice is controlled from within the phone’s “Speech” application. For the Nokia voices, the speed is not configurable.

Change Spelling Speed

This option increases or decreases the speed of the speech output (-10 to 10) when spelling a word with the spelling commands (e.g. “Command Key” + 8, in the default layout, or the corresponding touch screen gesture -- see the section with the overview of Command Shortcuts for a further description). The default level is 0 for the Normal and Beginner MS Profiles and 5 for the Advanced MS Profile.

Pitch

This option increases or decreases the pitch of speech output (-10 to 10). Default level is 0 for all MS Profiles. This default is already the recommended pitch for each voice. Note that the pitch settings have no effect when using the Loquendo, Acapela HQ, Nokia or Nokia HQ voices.

Voice on Call

This option allows the user to choose either the Nokia or (if available) the Nokia HQ TTS as the active voice while on a call. Note that in Samsung phones, it is not currently possible to hear speech output while on a call.

Volume on Call

This option increases or decreases the volume of speech output of Mobile Speak during a call (1 to 10). Default level is 8 for all MS Profiles.

Silent on Call

The “Silent on Calls” feature instructs Mobile Speak not to provide speech output while the user is on a call. If this option is off, then Mobile Speak works normally during a call and the user can navigate through the phone to, for example, check the calendar or the contacts list. Default setting is OFF for all MS Profiles.

Mute On Keypad Lock

The “Mute on Keypad Lock” setting affects the operation of Mobile Speak when the keypad is locked. When set to ON, Mobile Speak will be automatically muted when the keypad is locked. Speech output will be enabled automatically when the keypad is unlocked. Default setting is OFF for all MS Profiles.

Silent While Dialing

For more privacy, enabling this option lets the user dial numbers without speech feedback announcing each number pressed. Default is OFF for all MS Profiles.

Change the Audio Profile

Select the Normal audio profile when using Mobile Speak with speech routed through the phone’s speaker. Select the Ear phone audio profile when using a headset or ear phone. Default setting is Normal for all MS Profiles.

Change the Speech delay (seconds)

This allows the user to specify a delay (in seconds) which is added when speaking. This can be useful inside some applications where Mobile Speak reacts faster than the screen is updated. The additional delay will ensure that the correct item is spoken. Default setting is “Default” for all MS Profiles.

Reduce minimum volume

With this setting, one can set the percentage with which the volume is reduced at low volume levels (1-3). Possible values range from 12.5% to 100%. The default value is 100% (i.e. no reduction).

Increase maximum volume

With this setting, one can set the percentage with which the volume is increased at high volume levels (8-10). Possible values range from 100% to 400%. The default value is 100% (i.e. no increase).

## Reading Settings

The following is a list of the Reading configuration options:

- Change the Keyboard Echo settings.

- Change the Punctuation settings.

- Change the Secret Editor Mode.

- Toggle the Phonetic Spelling mode on/off.

- Toggle the Case Indicator.

- Toggle “Speak Deleted Characters”.

- Change the Number Processing settings.

- Change the Minimum number of processing digits

- Toggle the User Dictionary on/off.

Keyboard Echo

This setting specifies whether characters and words are spoken while entering text. There are four options:

- 	None: No speech feedback is provided while entering text.

- 	Characters: As the text is being typed, each character is spoken.

- 	Words: As the text is being typed, characters are not spoken, however as soon as a space character is inserted, the preceding word is spoken.

- 	Characters and words: Both characters and words are spoken while entering text.

It also has a related shortcut “Command Key” combination (“Command Key” + 9 in default layout, “Command Key” + Long press 9, in reading layout), as well as a corresponding touch screen gesture. See the Section with the overview of Command Shortcuts for a further description. Default setting is “Characters and Words” for all MS Profiles.

Punctuation

This option changes the punctuation setting (None / Some / All). It also has a related shortcut “Command Key” combination (“Command Key” + 7 in default layout, “Command Key” + Long press 7 in reading layout), as well as a corresponding touch screen gesture. See the Command Shortcuts section for more information. The default level is “Some” for all MS Profiles.

Secret Editor Mode

This setting controls the type of speech output when entering characters in a secret editor (e.g., the PIN code screen and password edit fields). When this option is set to “Asterisk”, the “*” character will be spoken each time a key is entered. When set to “Character”, the entered character is spoken. When set to “None”, there is no speech feedback while typing. It also has a related shortcut “Command Key” combination (“Command Key” + Long press * in all layouts), as well as a corresponding touch screen gesture. See the Command Shortcuts section for more information. Default setting is “Asterisk” for all MS Profiles.

Phonetic Spelling Mode

The phonetic spelling mode controls the spelling of letters (for example, after pressing “Command Key” + 8 in the default layout). Instead of spelling the letters by their names, they are spelled by means of the international radiotelephony phonetic alphabet (e.g., “a” is spelled “alpha”, “b” is spelled “bravo”, “c” is “charlie”, etc.). In general, vowels with accents will be interpreted as standard vowels. Nevertheless, in some special cases (e.g., Scandinavian vowels like Æ, Å, the Spanish consonant ñ, and the Czech letters with carons like ŭ or č) the characters are not spelled phonetically but just by their names. The default setting is “OFF” for the Normal MS profile, “ON” for the Beginner MS profile, and “OFF” for the Advanced MS profile.

Case Indicator

The “Case Indicator” setting changes the way capital letters are spoken in text editors (such as SMS, email, and help). When set to ON, the case of the character will be spoken when entering text with keyboard echo set to “Characters” or “Characters and Words”, when navigating text by character, and when spelling the current word (“Command key” + long press 6 or “Command Key” + long press #). The word "Caps" will be spoken immediately before the character if it is in uppercase. The default setting is “OFF” for the Normal MS Profile, “OFF” for the Beginner MS Profile, and “ON” for the Advanced MS Profile.

Speak Deleted Characters

When the Speak Deleted Characters option is set to “ON”, if you erase text in an editor, the deleted character(s) will be spoken. If verbosity level is set to High, it will also say "Deleted" followed by the character(s) that were deleted. Default setting is “OFF” for the Normal MS Profile, “ON” for the Beginner MS Profile, and “OFF” for the Advanced MS Profile.

Number Processing

This option controls the way that numbers are spoken by the TTS, and is applied to all numbers in all applications. Numbers containing less than the number of digits specified in “Minimum Number of Processing Digits” are sent directly to the TTS. However, numbers longer than this are processed according to the “Number Processing” setting. Available options are Single, Pairs, Triples, or TTS. The “Single”, “Pairs”, and “Triples” settings speak numbers in groups of one, two, and three digits respectively. The “TTS” option sends the number directly to the text-to-speech engine, which will speak the numbers according to its own rules. The Default setting is Single for all MS Profiles.

Change the Minimum Number of Processing Digits

This setting reflects the minimum number of digits that a number must have before the current Number Processing option is applied. The default value is 5 for all MS Profiles.

Toggle the User Dictionary On and Off

With this setting, the user can switch the user dictionary on or off.  This is identical to the “Command Key” shortcut when pressing “Command Key” + 4, in the default layout. For more information about the User Dictionary, see the end of this section. The default value is “ON” for all MS Profiles.

## General Settings

Below is a list of the General Settings:

- Change the Profile Name.

- Toggle the “Auto Start Mobile Speak” option.

- Toggle the “Power saving” mode.

- Toggle the “Welcome sound” setting.

- Toggle the “Allow shortcuts when muted” setting.

- Toggle the “Include in Toggle Profile Command” setting

Profile Name

Each MS Profile can be renamed according to your preference. For example, to help you remember which profile contains the settings you use when reading text messages, documents or web pages, rename that profile to “Reading” and this will be spoken when you use the associated command key to switch to a profile. Entering a new name here will change the name presented in the initial screen of the Mobile Speak configuration panel. Default names are Normal, Beginner, and Advanced.

Auto Start Mobile Speak

This option controls whether Mobile Speak will be launched when the phone starts. Note that if Mobile Speak is not loaded when the phone is restarted, you may need sighted assistance to navigate to the “Start Mobile Speak” shortcut, unless you have an assigned shortcut in the Standby Screen. Default setting is ON for all MS Profiles.

Power Saving

When this option is enabled, the backlight on the phone is turned off. This helps significantly in reducing the power consumption of the phone. Default setting is “OFF” for all MS Profiles.

Welcome Sound

The Welcome Sound setting can be used to activate and deactivate the sound and the animation displayed when Mobile Speak is launched. Default setting is “ON” for all MS Profiles.

Allow Shortcuts when Muted

When this option is set to “ON”, Command shortcuts (such as “Command Key” + 1, in default or magnification layout, to tell the date and time) will be performed even when the keypad is locked. If the setting is “OFF”, you must first unlock the keypad before using any of these commands. The default setting is “OFF” for the Normal MS Profile, “OFF” for the Beginner MS Profile, and “ON” for the Advanced MS Profile.

Include in Toggle Profile Command

When set to Off, the MS Profile is not included when toggling profile with Command + Clear. Default setting is “ON” for all MS Profiles.

Touch Mode (Touch phones only)

This setting allows the user to choose the touch mode that Mobile Speak will use. Available choices are: Keypad mode, Joystick mode, Quadrant mode, Review cursor mode and Stylus mode. It also has a related shortcut “Command Key” combination (“Command Key” + volume up, “Command Key” + volume down). Default setting is “Keypad mode” for all MS Profiles.

## Verbosity Settings

Below is a list of the Verbosity settings:

- Toggle verbosity of “List numbers” on/off.

- Toggle verbosity of “Tab numbers” on/off.

- Toggle verbosity of “Grid numbers” on/off.

- Toggle verbosity of “Menu key options” on/off.

- Toggle “Auto case change”.

- Toggle verbosity of “Character deletion”.

- Toggle verbosity of “Battery signal always” option on/off.

- Toggle verbosity of “Extended battery/signal” on/off.

- Toggle verbosity of “Submenu indicators” on/off.

- Toggle verbosity of “Announce window” option on/off.

- Toggle verbosity of “Announce unread messages” option on/off.

- Toggle verbosity of “Key feedback” on/off.

- Toggle “Speak marked at start” on/off.

List Numbers

This determines whether the index number of an item and the total number of items will be spoken in a list display, such as the list mode of the main phone menu (for example, “Tools, 5 of 17”). Default setting is “ON” for the Normal MS Profile, “ON” for the Beginner MS Profile, and “OFF” for the Advanced MS Profile.

#### Tab Numbers

This determines whether the index number of the tab and the total number of tabs will be spoken in a tabbed dialog (for example, “Speech, 1 of 6”). Default setting is “ON” for the Normal MS Profile, “ON” for the Beginner MS Profile, and “OFF” for the Advanced MS Profile.

#### Grid Numbers

This determines whether the index number of the icon and the total number of icons will be spoken in a grid display, such as the default mode of the main phone menu  (for example, “Tools, 5 of 17”). Default setting is “ON” for the Normal MS Profile, “ON” for the Beginner MS Profile, and “OFF” for the Advanced MS Profile.

#### Menu Key Options

This determines whether the functions assigned to the left and right soft keys (and also the Enter key, on 9.3 devices) are spoken (for example, “Button 1 select, button 2 exit, button 3 open”). Default setting is “ON” for the Normal MS Profile, “ON” for the Beginner MS Profile, and “OFF” for the Advanced MS Profile.

#### Auto Case Change

This determines whether to announce the change from upper-case to lower-case mode when the text input mode is set to “auto-case” (auto-case mode is indicated on the display with the icon “Abc”, and automatically capitalizes the first letter of each sentence while entering text, then switches back to lowercase input for the rest of the sentence). The default setting is “ON” for the Normal MS Profile, “ON” for the Beginner MS Profile, and “OFF” for the Advanced MS Profile.

#### Character Deletion

This determines whether to speak “deleted” when a character is deleted with the Clear key. The default setting is “OFF” for the Normal MS Profile, “ON” for the Beginner MS Profile, and “OFF” for the Advanced MS Profile.

#### Battery/Signal Always

When this option is enabled, Mobile Speak always speaks the time, battery level, signal level and operator when entering into the standby screen from a different application. This is a convenient way to check these values without needing to use a Mobile Speak Command (so it can easily be done for example by switching out and back into the standby screen, even on slider phones like the N86, while keeping the slider closed).The default setting is “ON" for the Normal MS Profile, “ON” for the Beginner MS Profile, and OFF for the Advanced MS Profile.

#### Extended Battery/Signal

When this option is enabled, Mobile Speak gives detailed information about battery and signal levels. For example, “battery level is 3 bars, signal level is 4 bars” as opposed to the less verbose “battery 3, signal 4”. The default setting is “ON” for the Normal MS Profile, “ON” for the Beginner MS Profile, and “OFF” for the Advanced MS Profile.

#### Submenu Indicators

When this option is enabled, Mobile Speak will say “Submenu” when a menu item has sub-item(s). Note that a submenu is opened or closed by pressing the joystick to the right or left respectively. The default setting is “ON” for the Normal MS Profile, “ON” for the Beginner MS Profile, and “OFF” for the Advanced MS Profile.

#### Announce Window

When this option is enabled, Mobile Speak will say “Window” when announcing the title of a screen. For example, “messaging window” vs. “messaging”. The default setting is ON for the Normal MS Profile, ON for the Beginner MS Profile, and OFF for the Advanced MS Profile.

Announce Unread Messages

When enabled, this option announces the presence of unread messages in the Inbox (e.g. “Inbox, unread messages”) when navigating through the messaging folder list, as well as indicating which messages in the Inbox are unread while scrolling through the message list. Default setting is ON for all MS profiles.

Key Feedback

When this option is enabled, additional feedback is given when pressing either of the soft keys, the joystick enter key, or the menu key. The default setting is “ON” for the Normal MS Profile, “ON” for the Beginner MS Profile, and “OFF” for the Advanced MS Profile.

Speak Marked at Start

This option allows the user to choose whether to speak "marked" at the start or at the end of any marked list or grid items. Default setting is ON for all MS profiles.

## Braille Settings

Below is a list of the Braille settings:

- Set Input Braille table.

- Set Output Braille table.

- Toggle “Show Control type” on/off.

- Toggle “Braille Cursor” on/off.

- Toggle “Auto turn off Bluetooth” on/off.

- Toggle “Control summary” on/off.

- Toggle “Input full pan” on/off.

- Toggle “Cursor blinking” on/off.

- Toggle “Suppress Capital Sign” on/off.

- Toggle “Reconnect at startup” on/off.

#### Input / Output Braille Table

These options set a Braille translation table to use for Braille input and output. There is a different table for each Braille grade and language. This table will be used when displaying text on the refreshable Braille line, or when entering text using the Braille keyboard. Beginning with Mobile Speak 3.0, you may select separate tables for input and output. For example, you may choose to enter text in US English grade 2 Braille, but have the output displayed in Spanish computer Braille. By default, 8-dot computer Braille tables are selected for both input and output in all MS profiles.

#### Show Control Type

When enabled, this option includes a short abbreviation before each text sent to the device, indicating the type of control being displayed. The Braille control type abbreviations vary according to the currently-selected language. With an English TTS, for example, ed is used to indicate that the control in focus is an editor. With a Norwegian TTS, on the other hand, sf (as in “skrivefelt” to refer to an editor) is used instead. The default setting is “ON” for all MS profiles.

#### Braille Cursor

In addition to the normal screen navigation mode, Mobile Speak has a “Braille cursor” mode, allowing you to view the contents of the screen in Braille, whether or not they are selectable using the system cursor of the phone, and to explore the screen without moving focus to another part or item. The default setting is “OFF” for all MS profiles.

#### Auto Turn Off Bluetooth

When this option is enabled, the Bluetooth capabilities of the phone will be automatically switched off when the Braille device is disconnected manually, either by terminating the connection between the mobile phone and Braille device, or by turning off the Braille device. This provides a higher level of security, and conserves battery power. Note that Bluetooth is automatically switched on when the connection between devices is established again. The default setting is “OFF” for all MS profiles.

#### Control Summary

This option, when used in conjunction with the Braille Cursor mode, adds an extra line for each editable control, selection list or grid, providing additional information about the location of the system focus within the control, as well as the number of lines and items in the control. For example, if system focus is on the second of four lines in an editor, the control summary displayed in Braille Cursor Mode will inform you - through a concise format suitable for short Braille displays - that the control is an editor with four lines and that focus is currently on line 2. The same applies to lists and grids, except that instead of lines, control summary provides details on the number of items and on which item is currently in focus. The default setting is “ON” for all MS profiles.

#### Input Full Pan

This option controls the scrolling behavior of the Braille line when the end of the line is reached while entering or editing text. If it is set to “ON”, the display will be scrolled by the entire width of the line such that the next character entered is shown in the first cell of a new Braille line. This setting is the default as most Braille users do not need to monitor the Braille display while typing. However, if it is set to OFF, the display will be scrolled by one character at a time so that the next character is shown near the end of the current Braille line. This setting is recommended if you want to keep track of what you have typed so far without relying on speech feedback.

#### Cursor Blinking

When this option is set to “ON”, the current position of the cursor within the text will be indicated on the Braille device via a flashing tactile cursor (dots 7 and 8 that go up and then retract every half-second). The blinking stops when the Braille device or mobile phone is not touched for 30 seconds. The default setting is “OFF” for all MS profiles.

#### Suppress Capital Sign

Enabling this option prevents the Braille output from indicating upper-case (“capital”) letters in Grade 1 or Grade 2 Braille by placing a dot 6 before the uppercase character. The default setting is “OFF” for all MS profiles.

#### Reconnect at Startup

When this option is enabled, Mobile Speak will automatically attempt to reconnect to the last-connected Braille device when the phone is restarted. The default setting is “OFF”.

## Magnifier Settings

Below is a list of the Magnifier settings:

- Change Zoom level.

- Toggle “Auto Start Magnifier” on/off.

- Change “Layout type”.

- Change the Lens size (Touch screen devices only).

- Change the Lens magnification (Touch screen devices only).

- Toggle “Auto zoom” on/off.

- Toggle “Scrolling” on/off.

- Change the Scrolling speed.

- Toggle “Border” option on/off.

- Change the Border Width.

- Change the Border Color.

- Toggle “Font smoothing” on/off.

- Change the Color Scheme.

- Toggle “Enhanced Color scheme” on/off.

- Toggle “Status shortcuts” on/off.

- Change the Distributed zoom level.

- Toggle “Text cursor” on/off.

- Change Text Cursor Color.

- Change the Cursor Size.

- Toggle “Cursor blinking” on/off.

#### Zoom Level

Eleven choices are available, ranging from 1x to 16x. The default setting is 2x for all MS Profiles.

#### Auto Start Magnifier

If this option is enabled, Mobile Magnifier will load automatically at phone startup. The default setting is “OFF” for all MS Profiles.

#### Screen Layout Type

Mobile Magnifier provides three different ways to magnify the screen:

- Full screen view (default layout for all MS Profiles): the contents of the entire screen are magnified.

- Distributed view: screen text is magnified in the center using the selected zoom level, while the window title at the top as well as current left and right soft keys at the bottom are displayed using a secondary magnification setting.

- Classic view: the bottom half of the screen shows what is in the unmagnified top half, but using a different magnification level.

Full Screen view

Distributed view

Classic view

#### Lens Size (Touch screen devices only)

This changes the size of the magnification lens that is opened when touching the screen for more than 1 second in Stylus mode (Small, Normal or Large). The default setting is “Normal” for the Normal MS Profile, “Large” for the Beginner MS Profile, and “Small” for the Advanced MS Profile.

#### Lens Magnification (Touch screen devices only)

This changes the zoom level of the magnification lens that is opened when touching the screen for more than 1 second in Stylus mode. Ten choices are available, ranging from 1.25x to 16x. The default setting is 2x for all MS Profiles.

#### Auto Zoom

Auto Zoom can be turned on or off. When turned on, the Magnification Level will automatically adjust to the amount of screen contents and may therefore change. Users who prefer to have a fixed zoom level should turn this feature OFF. The default setting is “OFF” for all MS Profiles.

#### Scrolling

When this option is enabled, if a high level of magnification is used and text does not fit the screen, Mobile Magnifier will automatically scroll to let you read the entire text without pressing any key. The default setting is “ON” for all MS Profiles.

#### Scrolling Speed

This option controls the rate at which Mobile Magnifier automatically pans the text when the “Scrolling” feature is enabled. There are 10 different scrolling speeds to choose from. Default setting is 3 for the Normal MS Profile, 1 for the Beginner MS Profile, and 5 for the Advanced MS Profile.

#### Border

Mobile Magnifier can draw a border outside the magnification area. The default setting is “OFF” for the Normal MS Profile, “ON” for the Beginner MS Profile, and “OFF” for the Advanced MS Profile.

#### Border Width

The width of the border can be customized to your preference. The default setting is “Small” for the Normal MS Profile, “Large” for the Beginner MS Profile, and “Small” for the Advanced MS Profile.

#### Border Color

The color of the border can be customized according to your preference (White / Red / Green / Black / Yellow / Blue). The default setting is “Red” for all MS Profiles.

#### Font Smoothing

Mobile Magnifier is the first solution for the Symbian platform to offer this very important feature for the visually impaired. Enabling this option smoothes the corners, curves and edges of the magnified screen contents to make them appear less jagged when enlarged and thus easier to read, even for extended lengths of time.

Here is an example:

Without font smoothing

With font smoothing

The default setting is “ON” for the Normal and Beginner MS Profiles, and “OFF” for the Advanced MS Profile.

#### Color Scheme

Mobile Magnifier has 7 different color schemes: Normal, Black and White, Grayscale (low resolution), Grayscale (high resolution), Color (low resolution), Color (high resolution), and Inverse Colors – meeting the needs of all users with varying degrees of vision impairment. The default setting is “Normal” for all MS Profiles.

#### Enhanced Color Scheme

When enabled, the Enhanced Color Scheme option will load a theme which has been specially designed to improve visibility of text. This theme is loaded into all applications, providing better readability of text. The default setting is “OFF” for all profiles.

#### Status Shortcuts

Enabling this feature gives you access to the following information screens:

Pressing the left soft key for half a second, the screen will show an icon indicating the battery level, network status and current time – all displayed with large icons and fonts.

Pressing the right soft key for half a second, the screen will show icons indicating if there are missed calls, unread messages and the date with large numbers.

You can enable or disable this feature using the “Status Shortcuts” setting. The default setting is “ON” for all MS Profiles.

#### Distributed Zoom Level

This is the secondary magnification level used for the title and soft key areas of the Distributed Views layout type. Default setting is 2x for all MS profiles.

#### Text Cursor

You can replace the default system cursor with your own customized text cursor - change its width and color, and turn blinking on or off. You can enable or disable this feature using the Text Cursor setting. The default setting is “ON” for all MS Profiles.

#### Text Cursor Color

You can customize the color of the text cursor (White / Red / Green / Black / Yellow / Blue). The default setting is “Red” for all MS Profiles.

#### Cursor Size

You can customize the size of the text cursor (Small, Medium, or Large). The default setting is “Large” for the Normal MS Profile, “Large” for the Beginner MS Profile, and “Small” for the Advanced MS Profile.

#### Cursor Blinking

You can control the blinking of the text cursor with this setting (ON or OFF). The default setting is “ON” for the Normal MS Profile, “ON” for the Beginner MS Profile, and “OFF” for the Advanced MS Profile.

## Maps Settings (only on phones supporting v3.03 or v3.04 of Ovi Maps)

The following is a list of the Maps configuration options:

- Automatically speak map changes.

- Walk/Drive Map changes spoken.

- Orientation type.

- Destination orientation.

- Distance limit.

Automatically speak map changes

This setting allows users to control whether or not to speak changes in the map's location or next turn automatically. There are three options:

- 	On: All changes will be spoken.

- 	When not in Walk or Drive mode: Will only speak changes in Map mode, not when navigating in Walk or Drive mode.

- 	Off: No changes will be spoken automatically. User should manually press an appropriate shortcut to find out the current location or next turn information.

The default setting is “On” for all MS Profiles.

Walk/Drive Map changes spoken

If map changes are set to be spoken automatically, this setting provides further control by allowing the possibility of limiting this automatic speaking while in Walk/Drive mode. There are four options:

- 	All: All changes will be spoken.

- 	When street changes: Will only speak when the street changes. Changes in distance only will not be spoken.

- 	When distance halves: In Walk mode: will automatically speak when the distance remaining till the next turn is half of what it was the previous time it was spoken.

- 	Use distance limit: In Walk mode: will automatically speak when the distance remaining till the next turn has changed by an amount greater than or equal to the distance specified in the “Distance limit” configuration option.

The default setting is “All” in the Normal and Beginner profiles, and "When distance halves" in the Advanced profile.

Orientation type

This setting allows users to configure how to speak orientation information with Ovi Maps shortcuts. There are two options:

- 	Compass: Speaks one of 16 compass directions. e.g. South southwest

- 	Degrees: speaks a value from 0-360 moving clockwise from North. So for example, West in Compass mode is equivalent to 270 degrees in Degrees mode

The default setting is “Compass” in all MS Profiles.

Destination orientation

This setting applies specifically to shortcut 8 in Walk/Drive mode. It controls whether the orientation to the target is spoken with respect to compass North, or with respect to the top of the map (corresponding to your heading), or both. There are three options:

- 	North: Speaks the orientation of the destination with respect to your current position as a Compass direction. It will be spoken as either “Compass” or “Degrees” values, depending on the setting of “Orientation type”.

- 	Map: Speaks the orientation of the destination with respect to the top of the map displayed on the screen (which in reality should correspond to your current heading once you start walking). It will be spoken as a Clockface direction (i.e. one of the 12 directions on a clock).

-	North and Map: Speaks both of the abovementioned orientations.

So for example, if you are in Walk mode and you press Shortcut 8 and “North and Map” is set, it may say “Distance to destination 545 metres, South East, 10 o’clock”. This will tell you how far remains, that the destination is at a compass direction South-East, and (assuming that the on-screen map is aligned with your walking direction), at an orientation of 10 o’clock from your current heading (i.e. ahead and to your left, at an angle of around 60 degrees from straight ahead).

Note that when in Walk mode, under the Options, some versions of Ovi Maps provide an option to "Align map to route", which is normally on by default. In this setting, the top of the map always faces in the direction we are currently being advised to walk. This means that the "Map" destination orientation (which is spoken here in clock face format) will be with respect to your current heading. It is very useful in this case to set the "Destination orientation" to "Map" or "North and Map", so that the direction received when pressing 8 will be with respect to the route's current direction. This setting may also be useful on devices which have a built-in compass and accelerometer to automatically adjust the map orientation, like the N8. In cases where it is not possible to align the map to your current heading, it is suggested to keep "Destination orientation" set to "North", so that the orientation is with respect to compass north and no misleading Map orientation information is given.

The default setting is “North and Map” in all MS Profiles.

Distance limit

This setting has an effect in Walk mode, if "Walk/Drive Map changes spoken" is set to “Use distance limit". In this case, “Next turn” changes are spoken only if the change in the remaining distance is greater than or equal to the selected distance limit. There are 7 options: 10 metres, 20 metres, 40 metres, 60 metres, 100 metres, 150 metres and 200 metres.

The default setting is 20 metres in the Normal profile, 10 metres in the Beginner profile, and 40 metres in the Advanced profile.

Note that when you are closer than 30 metres to the next turn, no distance limit is imposed, so all changes are spoken.

## Options under the Left Soft Key

When the left soft key is pressed (or the left soft key touch screen gesture is used) inside the Configure Mobile Speak application, a number of options appear. The list of all possible options is shown below. Note that some of the options only appear when the left soft key is pressed when configuring an MS Profile, while others only appear when you are viewing the list of all MS Profiles.

#### Change

This option is available when configuring a particular MS Profile setting. A new screen appears where you can change the currently highlighted item. Note that for items that only allow ON/OFF settings, pressing “Change” will toggle the setting instead of opening a new screen.

#### Apply to All Profiles

This option is available when configuring a particular MS Profile setting. It applies the setting of the currently highlighted item to all MS Profiles. For example, if the currently highlighted item is Volume and it is set to 4, then by pressing “Apply to All Profiles”, all MS Profiles will be set to have a volume of 4. Note that the only exception to this is the Profile Name setting. Pressing “Apply to All Profiles” has no effect on this setting, because it is confusing to have two profiles with the same name.

#### Activate profile

This option is available when viewing the list of all MS Profiles (either by pressing the left soft key or by pressing joystick enter, or the equivalent touch screen gesture). The MS Profile that is currently selected will be set as the active profile.

#### Customize profile

This option is available when viewing the list of all MS Profiles (either by pressing the left soft key or by pressing joystick enter, or the equivalent touch screen gesture). The configurations of the MS Profile that is currently selected will be opened, so that they can be viewed and modified.

#### Manage profiles submenu: Add new profile

This option is available when viewing the list of all MS Profiles. A new MS Profile will be created and added to the end of the MS Profile list. You will first be prompted to enter the name of the new profile. Apart from the name, this new profile’s settings will be identical to those of the currently selected profile.

#### Manage profiles submenu: Delete profile

This option is available when viewing the list of all MS Profiles. The currently selected profile will be deleted. You will first be prompted to confirm that you wish to delete the profile. Note that it is not possible to delete the profile that is currently active.

#### Manage profiles submenu: Assign to application

This option is available when viewing the list of all MS Profiles (either by pressing the left soft key or by pressing joystick enter, or the equivalent touch screen gesture). It allows you to assign the currently selected MS Profile to one or more of the applications (both built-in and 3rd-party) which are installed on your phone. When this option is selected, a list of all available applications on your phone will be listed alphabetically. Scroll through the list, marking any application which you would like to assign to the currently selected MS Profile by pressing joystick enter or the equivalent touch screen gesture. After selecting an application, note that it will say “marked” when you scroll through that item once more. Note that since an application can only be assigned to 1 MS Profile at a time, if you attempt to assign an application that has previously been assigned to a different MS Profile, you will be prompted for confirmation of this when saving the information (if you agree, then the old application assignment will be removed and the new one will be used instead). This is a very powerful feature of Mobile Speak, as it provides a convenient way to customize the way Mobile Speak and Mobile Magnifier work within particular applications.

#### Product Activation submenu: By SMS

This option will activate all installed Code Factory products (both trial and full licenses) by SMS. If no activation is performed, Mobile Speak or Mobile Magnifier will only work for 10 minutes. Please refer to the chapter on Licensing for more information about activating your Code Factory products.

#### Product Activation submenu: By Internet

This option will activate all installed Code Factory products (both trial and full licenses) by Internet. If no activation is performed, Mobile Speak or Mobile Magnifier will only work for 10 minutes. Please refer to the chapter on Licensing for more information about activating your Code Factory products.

#### Command Key submenu: Command Key 1

The “Command Key 1” submenu item of the Command Key item allows you to configure which key acts as the Mobile Speak Command Key, in order to access all the Command Key shortcuts in Mobile Speak.

The following list of possible keys is displayed:

- Default

- Menu key

- Send key

- Power key

- Multimedia key

- Edit key

- Gallery key

- Camera key

- Chr key (i.e. the key which will pop up the Symbol Table, which is available on many QWERTY phones)

- Calendar key

- Email key

- Contacts key

- None

As this is a checkbox list, you must scroll up or down to the selected key and press the joystick enter or the equivalent touch screen gesture in order to mark a key. After marking the desired key, press OK. If you simply scroll to a key and press OK without pressing Enter, the key will not be marked and the selection will not change.

The “Default” key is the edit key on devices where the edit key is available. If the device does not have an edit key, another appropriate key will be used as the “Default” Command key. The following list shows the “Default” Command key for each supported device.

- Edit key: 3250, 5500, E50, E60, E65, E70, N71, N73, N75, N76, N77, N80, N91, N92, N93, N93i, N95, N95 8GB, Samsung L870

- Multimedia key: 5320, 6290, N78, N79, N81, N82, N85, N96

- Calendar key: E51, E52, E55, E63, E66, E71, E72, E73, E75

- Camera key: 5228, 5230, 5530, 5630, 5730, 5800, 6120, 6121, 6124, 6220, 6700 Slide, 6710, 6720, N97, N97 Mini, C6, X6, Samsung i450

- GPS key: 6110, 6210, 6650 AT&T, 6730

- Left shift key: E61, E61i, E62

- Record key: E90

- Rewind / Fast Forward key: 5700

- Messaging key: 6790 Surge, E5

- Menu key: 5250, N86, C5, C5-03

- Music / Radio key: Samsung i550, Samsung i7110, Samsung G810

- Gallery key: Samsung i8510 INNOV8

Note that not all of the keys shown in the Command Key list are available on all devices, so make sure to select an appropriate key for your particular device. If a key that is not available is selected as the Command Key, it has the same effect as if “None” were selected.

The initial setting for Command Key 1 is “Default”.

#### Command Key submenu: Command key 2

Mobile Speak allows a second Command Key to be configured. This can be particularly useful in the case of flip or slider phones like the E75 or E90, where it may be convenient to have different Command keys with the keyboard closed or opened. The list of keys that can be selected and the general usage is identical to that of Command Key 1 – please see the previous subsection for a full description.

The initial setting for Command Key 2 is “None”.

#### Command Key submenu: Command Key Timeout

With this setting, the user can set the maximum time between pressing the “Command Key” and the subsequently pressed key to be still regarded as a Mobile Speak command. The default value is 3 seconds.

User Dictionary

Please refer to the "User Dictionary" section below for more information on this feature.

#### Braille Submenu: Configure Devices

The “Configure Devices” submenu item of the Braille menu opens the Braille device manager, which presents a list of the currently paired Braille devices. You can add additional devices to this list, delete a device, view information about a device, or connect to the currently selected device.

To add a Braille device to the list:

- Select “Add new device” from the Options menu opened with the left soft key.

Mobile Speak will present you with a list of supported devices.

- Select your device from the list.

The Device Manager will perform a Bluetooth search to establish a connection. Once the device has been configured, you may connect and disconnect it by selecting “Connect” from the options menu, or by using the Command + Dial key shortcut. Note that you can add more than one Braille device in the Device Manager, so switching between Braille devices is possible.

To delete, show information, or connect to a device, select the corresponding entry from the device manager list, and select the appropriate command from the Options menu.

#### Braille Submenu: Turn Braille on/off

This submenu item of the Braille menu performs the same function as the Command + Dial key combination. It will either connect to the currently configured Braille device (if not connection is currently active), or disconnect from the device if a connection is active. If no Braille device has been configured, an appropriate information message is displayed.

#### Restore Defaults

This option will have a different function depending upon where focus is when it is selected. If it is pressed while customizing an MS Profile, then all the settings for the MS Profile being customized will be restored to their defaults. However, if this option is pressed from the initial screen of the Mobile Speak configuration panel (i.e. the screen with the list of MS Profiles displayed), then all of Mobile Speak’s factory default settings will be restored. In both cases, the user will be prompted to confirm in order to proceed.

#### Help

A description of the shortcut to enter keyboard training mode is provided in the Help menu.

#### About

The date and version of Mobile Speak, copyright information, as well as the status of licenses (for every Code Factory product installed on the phone), the SIM status (if it is a trial or a phone-number based license and the original SIM is not in place) and the Device Serial Number (SN) will be spoken.

## TTS and Voices

Mobile Speak supports voices from several different text-to-speech technology providers, including Acapela, Fonix, and Loquendo. You can install as many voices for as many languages as you want, provided there is enough space on your device's memory or storage card to accommodate the TTS files. It is strongly advised that you always install TTS voices to the storage card in order to have as much free space in the device's main memory for running programs and storing data that cannot be moved to the storage card.

For each TTS voice installed, you can configure different parameters including the rate of speaking, pitch and volume, as well as a user dictionary for pronunciations that will be used if that voice is selected. For more information about configuring these and other parameters, please see “Configure Mobile Speak” in this manual.

An important new feature in Mobile Speak 4 is the ability to use up to three languages from a given TTS provider with only a single license. In other words, if you purchase a license for Mobile Speak with the Acapela TTS, you can use up to three Acapela voices for the price of one. This can be useful if you prefer different voices for different types of applications, or if you would like to install voices in different languages and always have them available. You can also uninstall a given voice and install a different one from the same TTS provider as often as you like without obtaining a new license.

If you would like to install more than three voices from a specific TTS provider, additional licenses can be purchased. Please contact your distributor for more information.

# Mobile Speak 4 Touch Interface

Mobile Speak 4 allows you to completely control your device using the touch screen. You can, of course, combine touch gestures and key commands in order to obtain the best experience using your device depending on its design and hardware features. It is always up to you to use touch gestures, keyboard shortcuts, or both.

Mobile Speak offers several different ways of interacting with the touch screen. These are referred to as “touch modes”. There are currently 5 different touch modes available. Two new modes have been introduced in v4.50: Keypad mode and Joystick mode. In addition, you may use Quadrant mode (which was the default in v4.0 and v4.01 of Mobile Speak), Review Cursor mode and Stylus mode. All of these touch modes are described in detail below.

In v4.50 of Mobile Speak, a new command has been added to allow users to easily change the touch mode. By pressing Command + Volume Down, the touch mode is changed to the next one in the list. Repeat this command several times until the desired touch mode is activated. Similarly, Command + Volume Up will activate the previous touch mode.

A new configuration setting called “Touch mode” allows the user to choose the touch mode that Mobile Speak will use in each Mobile Speak profile. Keypad mode is used as the default in all Mobile Speak profiles.

When inside some touch modes (specifically: Keypad and Quadrant modes), interaction with the touch screen can be done with navigation gestures as well as with taps on the screen. With a little practice, these gestures can be easily mastered and will become second nature.

For example, to simulate the joystick keys in these modes, a slide up/down/left/right should be performed. To perform a slide, touch the screen and slide your finger over it in the desired direction, then release the screen. Touch movements should cover a distance of about 30% of the screen length or greater, but do not need to span the entire width or height of the display. Slides should also be performed around the center of the screen, since many phones have a non-touch-sensitive area around the edges of the device. Note also that you do not need to slide very fast; it should take between a half and one second to slide the whole screen width or height.

## Keypad Mode

Mobile Speak v4.50 includes this new touch screen interaction mode, based on the concept of a standard numeric keypad. In this 12-key Keypad mode, the touch screen is divided into 12 virtual keys, arranged just like a phone keypad (1, 2, and 3 at the top; 4, 5, and 6 in the second row; 7, 8, and 9 in the third; and the star, 0, and hash keys in the bottom row). By accessing the Keypad mode, you can type numbers and letters just as with a physical keypad, including using predictive text to compose text messages and email. This input method allows you to enter text faster than either a virtual keyboard or a review cursor with an onscreen keyboard.

In addition, you can combine the numeric keypad keys with the command key to perform command shortcuts, just as in previous (non-touch) versions of Mobile Speak. For example, to mute Mobile Speak, you could press the command key followed by pressing the virtual 0 key at the center bottom of the screen. Longtime users of Mobile Speak will find this method of accessing commands familiar, and new users may also find it a convenient alternative to the standard touch gestures in Quadrant mode.

Besides the simple taps of number keys, the Keypad mode also includes a short list of gestures to access all of the navigational features of the phone. Moving left, right, up or down can be done with a simple slide in the corresponding direction. If you are in a list and want to move to the next item, for example, you can perform a slide down on the touch screen. If you keep your finger pressed against the screen at the end of the slide, the gesture will repeat, allowing you to move quickly and easily through long lists or menus.

To access the left and right soft keys, delete characters, or press enter, you can use a gesture called a "slide back" in one of four directions. For example, to access the left soft key, you can perform a "slide left and back" by sliding your finger right-to-left and then left-to-right without releasing your finger or pausing between the two motions. This gesture may take a few tries to be able to perform it smoothly, but once you familiarize yourself with it, it provides a quick and easy gesture to access common functions.

Keypad mode is activated by default when you first install Mobile Speak. If you would prefer to use a different touch mode, it can be changed in the “Touch mode” setting inside the General tab of Configure Mobile Speak. You can also change the touch mode using Command + Volume Up/Down.

Here is a complete summary of the command assignments in the Keypad mode:

| slide up* | move cursor up |
| --- | --- |
| slide down* | move cursor down |
| slide left* | move cursor left |
| slide right* | move cursor right |
| slide down and back | enter/press joystick |
| slide up and back* | delete/backspace |
| slide left and back | left soft key |
| slide right and back | right soft key |
| screen tap | keys 1-9, *, 0, # |

* = these commands can be held down at the end in order to repeat the assigned key.

In order to make it easy for you to get used to these commands, you can activate Mobile Speak Keyboard Help by pressing Command + Long tap 5. Once inside Keyboard Help, you can practice all the different taps and slides, so that you become familiar with the exact borders of your touch screen and the virtual keys and also with the slide gestures that you will use to interact with the phone. We strongly recommend that you spend a few minutes in Keyboard Help mode. Once you master the above list of taps and gestures, you'll be able to rapidly interact with your phone in all situations, including writing of text messages using T9 input.

## Joystick Mode

In the Joystick mode, the touch screen is divided into 9 virtual keys, arranged in a 3 x 3 grid. The central key is Joystick Enter, while the 4 keys directly above, below, left and right of the central key correspond to the Joystick up, down, left and right keys. The bottom left key is the left soft key, and the bottom right key is the right soft key, and the upper right key is the delete key (the upper left key is unassigned). This mode is intended for users who prefer to interact with the phone using taps rather than by slide gestures. It allows easy navigation through most of the phone's features. When typing a message or dialing a number however, it will be necessary to change to Keypad mode (or to activate the virtual keyboard from Quadrant mode). This can be done easily with Command + Volume Down (and then press Command + Volume Up to return to Joystick mode).

## Quadrant Mode

In this mode, the touch screen has been divided into 4 virtual keys named 1 to 4 clockwise and starting on the upper-left corner on the screen. This will sound familiar to users of previous versions of Mobile Speak Pocket. Imagine that you have 4 keys on the touch screen, and if you press the upper left corner area you are pressing the virtual key 1. The upper right part of the touch screen is the virtual key 2, virtual key 3 is placed near the bottom right corner and, the bottom left part of the touch screen corresponds to the virtual key 4. Touching a quickly releasing a virtual key is referred to as a “tap”. For example, tap 1 and hold will show you the Start Menu/Main Menu of the device, while tap 2 and hold closes the application you are in.

Some commands have been grouped in order to give you intuitive access to them. This way you have 3 secondary command groups named "Speak", "Go-to", and "Settings" command layouts.

As its name indicates, the Speak command layout contains commands which will make Mobile Speak give information to you (read date and time, read device status, repeat last spoken text) and reading commands.

The Go-to layout allows you to launch applications, press the soft-keys, move to previous or next control, launch special tools like the touch keyboard or the application switcher, etc.

The Settings layout allows you to configure Mobile Speak, mute, speech rate, volume, keyboard echo, etc.

These 3 secondary command layouts are accessible after the gestures:

- Tap 1: brings up the Go-to command layout.

- Tap 2: brings up the Settings command layout.

- Tap 4: brings up the Speak command layout.

For example, to have Mobile Speak read the device status you should use the command double tap 2 in the Speak layout. To do this, you first need to activate the Speak layout with tap 4. Mobile Speak will say "Speak", indicating that you have activated your desired layout. Then double tap 2 (read device status) and you will get the desired information.

Another example: imagine that you want to have Mobile Speak read the soft keys. To do that just tap 4 (Speak) and then tap 1, which is the gesture associated with the read soft keys command under the Speak layout.

In general, you can perform any command with at most two touch gestures. In addition to this, some gestures are directly available in the General layout.

Note that when you enter into a secondary command layout, it is possible to have different functions associated with the directional slides (more information can be found in the full command description section). For example, if you tap 2, bringing up the Settings command layout, sliding up and down will respectively increase and decrease the volume and sliding left and right will modify the speech rate.

Once you bring up a secondary command layout there are two ways to go back to the Quadrant mode’s General layout. There's a timeout which will send you back to the General layout after 10 seconds of inactivity, or alternatively you can triple tap the screen.

Triple tap is an excellent way to reset, or "return to normality" in most cases when in Quadrant mode. If you get lost, simply triple tap the screen to return to the General layout. Moreover, triple tap will also allow you to turn off mute, exit the review cursor, exit the virtual touch keyboard, and exit the command list.

Another useful feature to remember is that, after performing the Speak Soft keys command by tap 4 (speak) + tap 1 (read soft keys) you have 5 seconds to directly activate the desired soft key by a single tap 4 (left soft key) or tap 3 (right soft key). During this time you won't need to press tap 1 (Go-to) and then the desired soft key.

For a full list of the Touch commands available in Quadrant mode, please refer to the Touch Commands chapter in Appendix A.

## Command List (Accessed from Quadrant Mode)

The command list is a useful way of using the phone if you do not wish to learn the touch screen commands as it allows you to reach the available commands through the touch interface. To activate the command list when in Quadrant mode, perform a slide and hold gesture (i.e., slide in any direction, then stop moving and keep your finger on the screen until the voice announcement is heard) and Mobile Speak will speak "Command List".

Once this happens, you can access the available commands by sliding up and down in the same way as if it were a menu. This mode lists the available commands for the current layout, so by default you have access to Open Start Menu/Main Menu, Go-to, Settings, Stop speech, Speak, etc; if you are in the "Speak" command layout then this menu will give you access to the speak specific commands like read date and time, read current screen, etc. Use the command list to perform commands if you do not remember the gesture sequence to reach the desired functionality. For example, you know that tap 4 is "Speak", but you do not remember where in the Speak command group the read date and time function may be. In this case you can tap 4 (Speak) and then slide and hold the screen to have access to the list of commands available on the Speak group.

The commands are listed sequentially so you can easily select the one you want. The available gestures when you launch the Command List are:

- Slide up/down: Move to the previous/next available command.

- Double tap (anywhere): Execute the current command.

- Triple tap (anywhere): Close the command list and return to the default gesture mode for the current command layout.

- You can also tap and hold and move your finger up and down around the screen in order to explore the list of commands.

## Virtual Touch Keyboard (Accessed from Quadrant Mode)

There are several methods for entering text using Mobile Speak. If your device includes a physical keyboard, this provides a convenient and efficient method. Additionally, you can use an external QWERTY or Braille keyboard. The Keypad mode also allows rapid entry of text, supporting T9 and predictive text functionality. In addition, Mobile Speak also includes an on-screen virtual keyboard which allows you to enter text using the touch screen.

The virtual touch input method works by directly entering the characters on the touch screen of the device. When you activate this input method, a virtual keyboard is enabled on the screen. Use the stylus or your finger to select a character, keeping contact with the screen as you move through the characters. When a character is selected, Mobile Speak will speak that character. To insert the character that is currently selected, just lift your finger or the stylus from the screen. Note that as soon as you release the character held at the tip of the stylus or finger, it is inserted. The stylus or your finger should therefore remain pressed to the screen until you find the character that you wish to insert.

To activate the virtual keyboard, enter into Quadrant mode and then use tap 1 + tap 4 and hold. To exit the virtual keyboard, triple tap anywhere on the screen.

There are five keyboard types in this input method: lower case, upper case, symbol, numbers, and phone keypad. You can change the current keyboard type using double tap 2 or double tap 3.

You can also choose between various input languages by using double tap 1 or double tap 4. The available languages depend on the TTS you have installed. For example, if you have a French and English voice installed, you will toggle between QWERTY and AZERTY keyboards.

While you are writing with the virtual keyboard, use tap 2 to backspace (you do not have to look for the Backspace key every time you want to delete a character) and tap 3 to perform the Enter key.

Sliding up/down/left/right allow you to navigate the text while you are working with the touch keyboard.

The following summary lists the commands that are available from the virtual keyboard:

Slide Left 	Previous Character

Slide Right	Next Character

Slide Up	Previous Line

Slide Down	Next Line

Tap 2	Simulate Back key

Tap 3	Simulate Return key

Double tap 3*	Set Next Keyboard Layout

Double tap 2*	Set Previous keyboard layout

Double tap 4*	Set Next Language, if more than one language is installed

Double tap 1*	Set Previous language, if more than one language is installed

Triple tap	Exit, Virtual Keyboard

* After performing these commands, one can use single tap anywhere to keep cycling the command assignment. Note that 5 seconds of inactivity will turn this mode off, or alternatively you can use triple tap to go back to the Virtual Keyboard.

## Review Cursor Mode

Using Review Cursor mode on touch screen devices allows you to navigate the phone interface intuitively, by moving your finger over the screen while Mobile Speak reads the contents as you touch them. To activate Review Cursor mode, repeatedly press Command key + Volume Up or Command key + Volume Down to move through the list of touch modes until the Review Cursor mode is enabled. To return to the original touch mode, repeat the same process until the desired mode is enabled. Alternatively, the current MS profile’s “Touch mode” setting can be set to “Review cursor mode”.

You can either move your finger around the screen and Mobile Speak will read what is under your finger, or you can move between the items which are on the screen with slides up, down, left and right.

- Double tap: select current item.

- Slide left/right: previous/next character.

- Slide up/down: previous/next line.

- Tap 1: previous word.

- Tap 2: next word.

- Tap 4: move to home.

Note that not all screens and dialogs are supported yet by the review cursor. If inside an application that is not yet fully supported, it is suggested to use one of the other modes (Keypad, Joystick or Quadrant) to navigate around the screen.

## Stylus Mode

In this mode, you can use the screen without having the Mobile Speak touch interface enabled. This is useful for sighted people who may need to use the device with or without Mobile Speak.

To activate Stylus mode, repeatedly press Command key + Volume Up or Command key + Volume Down to move through the list of touch modes until the Stylus mode is entered. To return to the original touch mode, repeat the same process until the desired mode is enabled. Alternatively, the current MS profile’s “Touch mode” setting can be set to “Stylus mode”.

Some touch command gestures are available even when Stylus Mode is on by pressing the Command key followed by the gesture. The command key will temporarily allow you to perform Mobile Speak touch gestures, and once the command key times out, Stylus Mode will work normally again

# Braille

Mobile Speak can provide output through electronic refreshable Braille devices. Both text and control information will be available in Braille as you navigate around the mobile phone environment. Furthermore, using devices with Braille input keys, you can create and edit documents with any of the currently supported Braille grades and languages.

Through the Braille tab found in Mobile Speak’s configuration utility, users can set up connections with any number of supported Braille devices and easily switch between two or more of these at any time. Please refer to the section on configuring Mobile Speak in this document for more details on the various Braille configuration options which are available.

## Getting Started

For some devices (including those from HumanWare, BAUM, and Handy Tech), the first step in using Braille with Mobile Speak is to pair the Braille display with your device. Please check the documentation for your Braille device to find out the passcode used to connect with other devices. This may be specific to the serial number of the Braille device unit. Otherwise, you may use any string of numbers, normally no shorter than four digits, like 1234 which is the default passcode used by most Bluetooth-enabled devices. All Bluetooth devices use an authentication number when users attempt to establish a connection between them, thereby securing that connection.

Once the Braille device is added in the Braille page, there is no need to repeat the steps required to set it up for use with Mobile Speak, and you can toggle Braille on/off at any time. You will only need to repeat the process when:

- You perform a hard reset.

- You delete the Bluetooth partnership between your Braille device and your mobile phone.

- You remove the Braille device from the Braille page.

- You completely uninstall Mobile Speak and reinstall it or upgrade to a new version.

- Something has changed with the Braille device that alters its Bluetooth address.

## Entering and Editing Text Using Braille

When focus lands on an edit control, Braille output automatically switches to edit mode so that Mobile Speak displays the text cursor by raising dots 7 and 8. At present, this tactile cursor is only available when 6- or 8-dot computer Braille is selected. If there is an uppercase character at the cursor position, the dots will alternate, meaning that dot 8 is raised while dot 7 is not. If text selection commands are used, dots 7 and 8 will both appear in all the cells of the selected text.

When typing text using Grade 1 or 2 Braille, remember that the text is not translated until you press Space or Enter, or perform a command that moves focus or displays a message. Keep in mind that the Braille grade used for input is not necessarily the same one set for Braille output, so use the appropriate commands for configuring input and output Braille grades in order to be certain.

You can use the backspace command assigned to the key combination specific to your Braille device while typing in whichever grade to delete the character to the left of the text cursor. Note that this command may be used even before the text typed in grade 1 or 2 Braille is translated. If you wish to clear everything you have typed so far that has not yet been translated, use the Braille Escape command available on all supported Braille devices.

# Magnification

Mobile Magnifier is a full-screen magnification application that enlarges and improves readability of screen contents, detects areas of interest to make them more visible, and offers hotkeys to obtain a magnified view of screen icons conveying non-textual information. With Mobile Magnifier you can:

- Enlarge screen contents in ten variable levels of magnification.

- Smooth curves and edges of magnified fonts and graphics.

- Access seven different color schemes for better visual contrast.

- Magnify the whole screen or just areas of interest.

- Easily pan and scroll screen contents even in magnified view.

Although licensed separately, Mobile Magnifier can be installed together with Mobile Speak, making it easy to provide speech output and screen magnification at the same time if desired. Note that in Symbian devices Mobile Magnifier is always installed together with Mobile Speak (on supported devices) and cannot be separately installed.

## Exploring the Mobile Phone Screen

Although Mobile Magnifier will automatically detect the area of interest and magnify it in real-time, there may be instances where freely exploring the entire screen is needed. In order to do this, you can press and hold the arrow keys at any time while Mobile Magnifier is running to pan the screen contents.

## Starting and Stopping Mobile Magnifier

Mobile Magnifier can be started or stopped in any of the following ways:

- Pressing the Command Key + Long 0

- From the Code Factory folder located under the phone’s Main Menu

- Tap 2 (Settings) + tap 4 through the touch screen when in Quadrant mode

## Configuring Mobile Magnifier

Mobile Magnifier provides a wide range of configuration settings to fit the needs of each user and provide the best user experience for each different phone. These settings can be configured from the Magnification Page found in the Mobile Speak configuration utility. Please see to the section on Magnifier Settings in Chapter 7 of this manual for more details. In addition, many Magnification settings can be adjusted via Command Key shortcuts while in the Magnification layout. To enter the Magnification layout, first make sure that the Magnifier is activated. Then press Command + 5 and loop through the layouts until the Magnification layout is activated. A full list of all the shortcuts available in the Magnification layout is available in Appendix A.

## Magnification Lens (Touch screen devices only)

As well as providing all of the Magnifier features offered on 3rd edition devices, on touch screen devices a new Touch Magnification Lens is provided. When Mobile Speak is in Stylus mode, you can press anywhere on the screen and within 1 second, a Lens will be displayed, magnifying the area around the point that you touched on the screen. As you move your finger around the screen, the Lens moves with you, magnifying the area of the original screen located directly below your finger. When your finger leaves the screen, the area where you last touched the screen remains magnified. This allows you to easily explore all areas of the screen in a rapid and intuitive way. Low vision users will love this new feature. There are two new configuration settings in the Magnifier tab on Touch devices which are related to the new lens: Lens size (small, normal or large, with normal being the default) and Lens magnification (1.25x to 16x).

# User Dictionary

You can change the way your current TTS voice pronounces words through Mobile Speak’s User Dictionary.

There are several ways to access the User Dictionary:

- Command key + Long 4

- In the Configure Mobile Speak application, press the left soft key and select ‘User Dictionary’.

When opened, the User Dictionary application shows a list of all the TTSs that you have installed on your phone. User dictionary tool options are:

- Add new word: adds a case-insensitive word to the user dictionaries of all the marked TTSs. For example, ‘Hello’ will be considered the same word as ‘hello’. This option consists of two steps: first, you have to enter the word to be replaced and then the replacement word. Note that you must first mark at least one TTS in order to add a new word; otherwise the message “No dictionaries marked” will appear.

- Add new case-sensitive word: adds a word to the user dictionary of all the marked TTSs differentiating them by the use of capital letter. Example: ‘Hello’ will be considered as different to ‘hello’. To add a new case-sensitive word, follow the same steps above. Note that you must also first mark at least one TTS in order to add a new word; otherwise the message “No dictionaries marked” will appear.

- Mark/Unmark: allows you to mark or unmark specific TTSs, so that operations such as “Add new word” will be performed on multiple TTSs. Note that pressing the Joystick Enter key toggles the Marked / Unmarked status of the current TTS.

- Show word list: shows all the words contained in the selected TTS. It appears as a list, but you can also do a quick search by typing the word (like in the Contacts application). This feature is useful when there are hundreds of words. When a word is selected it can be deleted or edited.

- Refresh dictionary list: updates the TTS list. This option can be useful, for example, if you have just installed a new user dictionary via the PC tool and the dictionary has not yet been automatically updated.

- Reset Dictionary: deletes the user dictionary of the selected TTS. Note that pressing the Clear key has the same effect.

- Copy Dictionary: Copied the currently highlighted dictionary, so that it can later be pasted to another TTS. Note that an empty dictionary cannot be copied.

- Paste Dictionary: Pastes the previously copied dictionary. This option is only displayed after the Copy Dictionary process has been performed.

- Help: speaks help information regarding the usage of the User Dictionary.

Now we also support the following punctuation signs when adding words to the user dictionary (for both case-sensitive or case-insensitive): ! " # $ % & ‘ ( ) * + , - . / : ; < = > ? @ [ \ ] [ ^ _ ` { } ~

Please note that there is no need to reload the current TTS in order to reflect changes when adding words. This is done automatically when a word is added or deleted.

Here are some general things to keep in mind when using the user dictionary:

- You can add phrases to the user dictionary; however it behaves differently than when adding a single word. Mobile Speak only replaces phrases that are exactly as defined in the user dictionary. For example: If we replace "Missed calls", then when speaking "2 Missed calls" it won't be replaced as this text is not exactly the same as the phrase in the dictionary.

- The user dictionaries are stored on the phone in files called userdict.dat. There is a different file for each TTS, and the location of each userdict.dat file depends on the TTS and voice that are used. The folder where the files are stored is in the format /data/TTS_Name/TTS_Language/[TTS_Voice/]. For example, the Acapela Spanish Javier user dictionary is stored at /data/speechacapela/Spanish/Javier/userdict.dat. Unless you have first copied a user dictionary to a different memory location, the files will always be created on the C: drive (i.e. phone memory). If you’d like to share your User Dictionary with other phones, you simply need to transfer the userdict.dat file to the other phones and place it in the same folder.

# Browsing the Web

The web browser is available on all Symbian 3rd and 5th edition phones. Mobile Speak provides easy to use and powerful access to web content via the Web browser, allowing you to navigate web pages in the way you are used to with your desktop screen reader. In most phones, this browser is called "Web". In 9.2, 9.3, and 5th edition phones, it is the default browser and the only one preinstalled on the device. It can also be launched by doing a long press of the “0” key from the standby screen. In 9.1 phones, it is not the default browser - the default browser is typically called "Services" and is also supported by Mobile Speak. If you have a 9.1 phone and want to use the newly supported Web browser, make sure that you are launching the correct browser (i.e. the one typically called "Web").

Mobile Speak provides support for the web browser by means of a virtual cursor. The default key press functionality has been changed in order to provide special functionality and allow easy navigation through the web page. While a page is loading, on most phones you can press “joystick up” to hear how much data has already been loaded and how much remains (e.g. 123 of 456 KB). Once a page is fully loaded, a page summary with the number of frames, headings, elements and links will be spoken. Once this is spoken, you can begin to navigate the page using the Mobile Speak virtual cursor. Note: you must wait until the page is fully loaded to start using the Mobile Speak virtual cursor.

Please keep in mind that you will need an Internet connection in order to use this web browser. If you do not connect via WLAN, please check the data plan of your operator so that you know exactly what charges are involved with downloading Internet data.

Below is a summary of the virtual cursor commands currently available:

- Joystick down: Move forward one element in the web page.

- Joystick up: Move back one element in the web page.

- Joystick right: Jump forward one element based on the current Jump Mode. The Jump Mode can be changed by pressing the keys 2-8 (see below). The default Jump Mode whenever a new page is loaded is "Jump by Links".

- Joystick left: Jump back one element based on the current Jump Mode. The Jump Mode can be changed by pressing the keys 2-8 (see below). The default Jump Mode whenever a new page is loaded is "Jump by Links".

- 0: Toggles between start and end of a page.

- 2: C for Control. Toggles the Jump Mode between: Jump to same control; Jump to different control; Jump to graphical control.

- 3: F for Forms. Toggles the Jump Mode between: Jump by forms; Jump by editors; Jump by buttons; Jump by checkbox and radio button; Jump by combo box and list box.

- 4: H for Headings. Toggles the Jump Mode between: Jump by headings and frames; Jump by headings; Jump by frames.

- 5: L for Links. Toggles the Jump Mode between: Jump by links; Jump by non-links; Jump by in-page links.

- 6: M for Move. Move forward or backward in the page by a fixed percentage of the page size (in elements). Options are: 2%, 5%, and 10%. Information about the current element number in the page is also provided to give an indication of your current location in the page.

- 7: P for Paragraph. Toggles the Jump Mode between: Jump by paragraph and block quotes; Jump by paragraph; Jump by block quotes.

- 8: T for Tables. Toggles the Jump Mode between: Jump by table and list; Jump by list; Jump by table.

- 9: Read to end of page. This will speak the contents of the page, starting at the current element. Press the * key to stop speaking, and the virtual cursor will be positioned at the element where you stopped.

- Clear: Move backwards in the current page using your navigation history. This provides a convenient way to trace back the steps as you navigate through a page.

- Send key: Opens a search window, allowing the user to enter a search term. The left and right joystick keys can then be used to repeatedly jump through and move to the next found element.

- Enter: Appropriate behaviour depending on the type of control. For example, when currently focused on a link, navigate to the link. When currently focused on a text entry box, open the editor so that text can be entered. Note that when pressing Enter when at the start of a list or table, it will now jump to the end of the table, and vice versa. This provides an easy way to rapidly pass over lists and tables without having to enter all their elements.

The left and right soft keys have their normal functionality in the web browser. The left soft key pops up a menu of available browser options, while the right soft key allows the user to move back to the previous page.

Note: JavaScript must be enabled in the Web Settings menu (it is enabled by default, so unless you have disabled it this should not be an issue).

# Third-Party Application Support

## QuickWord

Opening a document with QuickWord

When QuickWord is opened, a navigable list of options appears. However, when selecting “Recent documents” or “C:Phone memory”, a list will appear with respectively, all recently opened documents, or the entire contents of the accessible phone memory. However, these contents are not currently accessible with Mobile Speak.

An alternative and accessible way to access the list of word documents is to use the phone’s File Manager, which on many phones is located in the “Tools” folder. Opening File Manager will show a list of files in the phone memory, and when scrolling through this list, the name of the selected item is spoken. When selecting a Word document, QuickWord will open this file.

Starting up QuickWord

When opening a .DOC file in the File Manager, the QuickWord program will open and the document will appear in the screen, in “Read Only” mode. This “read only” view is not accessible with Mobile Speak. However, when selecting Options -> Edit Document, the text will be in editing mode and will be fully accessible. If your version of QuickWord does not allow editing, you may need to purchase a full license of QuickWord in order to be accessible with Mobile Speak.

When in editing mode, the document’s entire text is spoken. This can be interrupted by pressing one of the arrow keys. From that point on, the normal Mobile Speak key commands work, just as in the case of other editor programs (e.g. text messages).

Reading a Document with QuickWord

When starting in editing mode, the entire text will be spoken. This is interrupted as soon as any key is pressed. Depending on which key is pressed, the corresponding command will be executed. The most powerful Mobile Speak commands are available from the “reading layout” (entered by pressing Command Key + 5). The most important reading commands are:

- Command Key + #. Speak the entire text.

- Command Key + Long Pressed #. Spell the current word.

- Command Key + Long Pressed Arrow Up. Speak the previous line.

- Command Key + Long Pressed Arrow Down. Speak the next line.

- Command Key + 1. Speak the previous sentence.

- Command Key + 2. Speak the entire text.

- Command Key + Long Pressed 2. Go to the beginning of the text.

- Command Key + 3. Speak the next sentence.

- Command Key + 4. Speak the previous word.

- Command Key + 6. Speak the next word.

- Command Key + Long Pressed 6. Speak the current word.

- Command Key + 7. Speak the previous paragraph.

- Command Key + 8. Speak from the current cursor position.

- Command Key + Long Pressed 8. Move the cursor to the end of the text.

- Command Key + 9. Speak the next paragraph.

Editing a Document with QuickWord

When inserting a new word, each letter is spoken out. After typing a space, the last inserted word is entirely spoken out. When the editor is in T9 mode, after pressing a new letter, the entire word is spoken out.

## Fring

Starting Fring

When starting Fring, it will attempt to connect to the Internet. If no connection is available, it will inform you. All necessary information during this process is spoken by Mobile Speak.

When successfully logged in, a list of contacts as added during the configuration or during previous settings will appear on the screen. When moving the cursor up and down, the highlighted contact will be spoken out.

Press the middle button to select the contact. The chat window will open.

Chatting with Fring

The chat window consists of two sections: the upper section is the chat section and the lower section is the text input section, where the cursor is always located. All the text that you type will appear in this section.

To send written text to the other party in the chat session, click on the center button (recommended). Alternatively, you can press the left soft key to open the Options menu and then select Send.

When the other party responds, a ringtone will be played. At this point, you have three possibilities to hear the received text. For all three possibilities, you must set the window to “Reading Layout” (by pressing Command Key + 5).

- Command Key + Long Pressed Arrow Down - speak the last received line.

- Command Key + Long Pressed Arrow Up - speak all previously unspoken text from beginning to end. If no new text has arrived since the last time this command has been pressed, it will speak only the previous line (the line above the line that was spoken the last time).

- Command Key + # - speak the entire text from the chat section.

To leave the chat window, click Options (upper left button) and then scroll down until you can select Close.

# Ovi Maps Support

We're happy to provide you with support for the latest versions v3.03 and v3.04 of Nokia Ovi Maps. This provides users with free, voice-guided turn-by-turn assistance in both pedestrian (Walk) and car (Drive) modes.

To learn about the full functionality of Nokia Ovi Maps, please visit the Ovi Maps support page and review the user guide and FAQs here.

## Supported Devices

For the most up-to-date list of supported devices, as well as information about required firmware versions etc, please visit the Nokia Ovi Maps device compatibility page here. Currently Nokia Ovi Maps v3.03 and v3.04 are supported on the following 3rd edition devices: E66, E71, N86, C5, E5, E52, E55, E72, E73, 6710, 6720, 6730, 6760, and also on the following touch devices: 5230, 5800, N97, N97 mini, C6, X6, and will also be supported on these Symbian^3 devices: N8, E7, C6-01, C7.

## Basic Operation of Ovi Maps v3.03 and v3.04:

The Maps application may be found in a different location, depending on the model of phone that you have. Upon opening the application, by default an attempt is made to connect to the internet. After a few seconds a grid menu appears. The names of these grid items are now accessible with Mobile Speak. The main items which are of interest and which are fully accessible are:

- My position: Opens a map showing your current location, which is positioned at the origin of the map. We refer to this screen as “Map mode”. Your current position is displayed on a label at the top of the screen and will be spoken. If configured to do so, Mobile Speak will automatically speak the position again if it changes (for example, as you walk around). Additionally, the shortcuts mentioned below will provide specific useful information. Initially the map's origin focuses on your current location, and the map will move to track your current location and keep it at the origin of the map. However, it is also possible to perform virtual navigation of the map by pressing the joystick directional keys (or by directly touching the map on Touch devices while in Stylus mode), which will move the map’s origin to the location that you choose. Once this has been done, the map will no longer track your current location as you move around, and the new map origin will be used as the point of reference for searches etc. This provides a useful way to virtually explore the map. By pressing 0, the map origin will revert to tracking your current location.

- Search (called “Find Places” on some devices): Opens an editor which will allow you to enter any search term (e.g. street, restaurant name, town or park). After entering your search term, press Joystick Enter and an on-line search will be made, bringing up a list of all items matching your search which you can browse and bring up on the map. Rather than entering a specific search term, if you navigate down 1 item from the editor you will find the "Browse categories" control, which brings up another list of search items by category (e.g. Transport, Accommodation, Shopping etc). Some of the main categories may have sub-categories to further narrow down the search. Clicking on any of these will bring up a list of locations which match your search criteria and are close to your current map origin. These can then be selected and found on the map, saved, or a Walking or Driving route to the location can then be created.

- Favorites: Allows you to bookmark your favorite places or routes, or to create your own collection of favorites so you can keep track of all your personal Points of Interest.

- Walk: Initially brings up a 3-item grid with options "Walk home", "Set destination" and "Map". Once the desired destination has been selected, a route will be created and displayed on the screen (and spoken by Mobile Speak). We refer to this screen as “Walk mode”. In addition, Ovi Maps can be configured to use the Nokia HQ TTS to speak information about the next turn (distance and direction to turn). By combining Mobile Speak's new Ovi Maps features with the built-in Walk Guidance, a very powerful, accessible GPS navigation solution is provided.

- Drive: We refer to this screen as “Drive mode”. This works in a very similar way to the Walk guidance. Once a destination is selected, voice-guided turn-by-turn assistance is provided. Mobile Speak provided various shortcuts to provide additional routing information.

- Ovi Maps configuration: From the main menu, press Options > Settings to configure Ovi Maps to your preference. The items found under "Navigation" are particularly useful, since they allow you to activate both Walk and Drive guidance and define your home location.

## Shortcuts when in Map, Walk or Drive Mode:

Ovi Maps comes with a number of predefined shortcuts when in Map, Walk or Drive mode. These are explained in full detail in the Ovi Maps User Guide. Mobile Speak provides additional shortcuts (using numeric shortcuts that were either not being used by Ovi Maps, or that were of limited use for visually impaired users). The most useful shortcuts (for both Ovi Maps and Mobile Speak) are listed below. We recommend users of Touch devices to use the Keypad mode in order to have easy access to these shortcuts:

- 1: In Walk/Drive mode: Mobile Speak shortcut to speak the next turn. In Map mode: Ovi Maps shortcut to change Map Type (Map, Satellite or Terrain).

- 2: In Walk/Drive mode: Ovi Maps shortcut to save the current position.

- 3: In Walk/Drive mode: Ovi Maps shortcut to search using the "Browse categories" list.

- 4: In Map mode: this Ovi Maps shortcut rotates the map anticlockwise. Mobile Speak will speak the new map orientation with respect to north (as a compass direction or in degrees, depending on Mobile Speak's "Orientation type" setting). In Drive mode: Ovi Maps shortcut to repeat last voice guidance given by Ovi Maps.

- 5: In Map mode: this Ovi Maps shortcut rotates the map so that the top of the map is facing north. Mobile Speak will speak the new map orientation (e.g. "North" or "0 degrees", depending on Mobile Speak's "Orientation type" setting). In Walk mode: Mobile Speak shortcut to speak the map's orientation with respect to north. Note that on phones with a built-in compass, the compass is activated/deactivated each time you press the 5 key.

- 6: In Map mode: this Ovi Maps shortcut rotates the map clockwise. Mobile Speak will speak the new map orientation with respect to north. In Walk/Drive mode: Ovi Maps shortcut which allows user to adjust Ovi Maps TTS volume up/down using the joystick up/down keys during 5 seconds.

- 7: In Walk/Drive mode: Ovi Maps shortcut to bring up list of route points.

- 8: In Walk/Drive mode: Mobile Speak shortcut to speak the total distance remaining to reach the destination, and also the direction of the destination (either with respect to north, with respect to the top of the map, or both, depending on Mobile Speak's "Destination orientation" setting). This extremely useful command allows the user to regularly check the distance to the destination, which may be particularly useful when a route is first started and the user is unsure if he/she is heading in the right direction. When in Walk mode, by default the top of the map always faces in the direction we are currently being advised to walk (which corresponds to your current heading). It is very useful in this case to set the "Destination orientation" to "Map" or "North and Map", so that the direction spoken when pressing 8 will be with respect to your current heading. Note that on devices without a compass (or with the compass deactivated) the map may take a few seconds to adjust correctly shortly before and after you make a turn, so it’s best to use this orientation information once you have walked a short distance in the same direction. This setting is also very useful on devices which have a built-in compass and accelerometer to automatically adjust the map orientation. With the compass activated, hold the phone flat in your hand so that the top of the phone is pointing in the direction you are walking, and the clockface orientation given will be with respect to your current heading.

- 9: In Walk/Drive mode: Ovi Maps shortcut to bring up the accessible "dashboard", which is a listing of distance, time and speed for the current trip and in total. By pressing "Options" either the current trip or the total information can be reset. In Map mode: brings up inaccessible list of Ovi Maps shortcuts in Map mode.

- 0: If map origin has changed by virtually exploring, this Ovi Maps shortcut will return the map origin to your current position. Mobile Speak will then speak the current position.

- Clear: In Map mode: Mobile Speak shortcut to speak the current map origin's position (this may either by your current position or your virtual position if you have started to move virtually around the map). In Walk/Drive mode: shortcut to speak your current position, regardless of whether or not the virtual map origin has changed.

- *: Ovi Maps shortcut to zoom in the map scale. Mobile Speak will then speak the approximate distance corresponding to a screen width.

- #: Ovi Maps shortcut to zoom out the map scale. Mobile Speak will then speak the approximate distance corresponding to a screen width.

- Joystick up/down/left/right: change the map origin, allowing you to navigate virtually around the map. Mobile Speak will speak the new origin's position as it is moved.

- Joystick enter: In Map mode: after navigating virtually to a new location, this will open up a menu allowing you to (amongst other things) create a route to walk to this location. Otherwise, it pops up a list allowing you to do things like save the place or provide more details about the current location.

- In addition, Mobile Speak's Command + # may speak some additional useful textual information found on the screen (such as distance from current map origin to current position when virtually exploring in Map mode).

## Maps Configuration Tab in Configure Mobile Speak:

On phones supporting Ovi Maps v3.03 and v3.04, a new tab called "Maps" has been added to the end of each Mobile Speak profile's configuration tab list. These items are described in detail in the “Maps Settings” section in Chapter 7.

## Known Issues with Ovi Maps v3.03 and v3.04:

- Map scale information may not be available on all devices when pressing * and # (especially in v3.04 of Ovi Maps).

- On Touch phones, dialing # in keypad mode while on a map may not zoom out as expected.

- In Walk/Drive mode: Ovi Maps and Mobile Speak may sometimes speak at the same time. If using a 3rd party TTS in Mobile Speak (e.g. Acapela, Fonix or Loquendo), both voices will be heard at the same time. However, if using the Nokia or Nokia HQ TTS in Mobile Speak, the Ovi Maps and Mobile texts will interrupt each other if one tries to speak before the other has completed its text. If this is an issue, you can prevent Mobile Speak from automatically speaking map changes inside the Mobile Speak configuration, so that any Mobile Speak speech will be fully controlled by the user.

- Features not supported: In the main Ovi Maps menu, there may appear several icons providing additional web-based services (e.g. Weather, Events, Lonely Planet and Michelin). These features are not accessible with Mobile Speak.

- As there are many different minor builds available for both v3.03 and v3.04 of Ovi Maps, we have not been able to fully test with all combinations of minor build and device. It is therefore possible that some build/device combinations may not provide the desired accessibility or may not be completely stable inside the Ovi Maps application. It is expected that any issues that may arise with certain build versions or with certain devices will be resolved in future releases of Mobile Speak.

# EULA

Mobile Speak, Mobile Magnifier and all other Code Factory products

Copyright (C) 2002-2010 Code Factory, S.L.

All Rights Reserved

LICENSE AGREEMENT

By installing or activating the Code Factory products and their speech, magnification, or Braille components, you accept all the terms and conditions of this license agreement:

1. GRANT.

Code Factory, S.L. (referred to hereafter as "Code Factory"), with its principal place of business at Rambla d'Egara, 148, 2-2, 08221 Terrassa, Spain, hereby grants to you a user-centered license to use the Code Factory products (referred to hereafter as "Software"), and accompanying product documentation (referred to hereafter as "Documentation") on the following terms:

2. PERMISSIONS.

You may:

(i) install, activate, and use the Software on any supported phone meeting the product's system requirements;

(ii) install and use the Software on another device provided that:

(a) you activated the Software on the second device with a trial license,

(b) you activated the Software on the second device using a full license different from the one used on the first device, or

(c) you have transferred your phone number to the device on which you wish to run the product.

(iii) copy the Software in machine-readable form solely for back-up or archival purposes, or use within a single working location, provided you reproduce Code Factory's copyright notice and proprietary legends.

3. RESTRICTIONS.

You may not:

(i) modify, translate, adapt, reverse engineer, decompile, disassemble, or otherwise reduce the Software to a human perceivable form;

(ii) create derivative works based on the Software or any portion thereof, or the Documentation, or attempt to increase the functionality of the Software in any manner except through scripting or other improvement functionality accessible to the user and supported by Code Factory.

(iii) copy the Software (except for backup purposes) or the Documentation. Copying includes, but is not limited to, reproduction through any electronic or mechanical means, and distributing copies;

(iv) remove any proprietary notices or labels on or in the Software or Documentation.

4. OWNERSHIP.

Code Factory retains the title, ownership rights, intellectual property rights, and trade secrets in and to the Software and Documentation, including all subsequent copies and updates to the Software and Documentation, regardless of the form or media used.

Similarly, the title, ownership rights, and intellectual property rights in and to the content accessed through the Software is the property of the applicable content owner and may be protected by applicable copyright or other law. This license gives you no rights to such content.

The Software is protected by the copyright laws of Spain and international copyright treaties. In no manner and under no circumstances is this license to be interpreted as a sale of the title, ownership and intellectual property rights in and to the Software and Documentation.

5. LIMITED WARRANTY.

Code Factory warrants that the Software will perform substantially on an officially supported device in accordance with accompanying written materials for a period of thirty (30) days with a trial license, or with no time limitations with a full license, providing that the version of the Software and the operating system and firmware version of the device on which it runs do not change. This limited warranty is void if failure of the Software has resulted from user error, accident, abuse, misapplication, and defects in the Software and hardware of the device. This is the sole and exclusive warranty which Code Factory makes as to its software. Some states/countries/jurisdictions do not allow limitations on duration of an implied warranty, so this limitation may not apply to you.

Except as expressly provided in the limited warranty section above, the Software is provided to the end user "as is" without warranty of any kind, express or implied. Without limiting the foregoing, any warranty of non-infringement of third parties' rights, as well as warranties of merchantability and of fitness for a particular purpose are specifically disclaimed. The entire risk of the quality and performance of the Software is with you.

No Code Factory dealer, reseller, agent, employee, or any other party is authorized to make warranties or conditions on Code Factory's behalf. You may have other legal rights that vary from state to state or by jurisdiction.

6. DISCLAIMERS.

You accept all risks which may arise from the downloading, installation and use of the Software including, but not limited to, errors in transmission, corruption of existing data or Software in the device, and/or damage to the hardware of the device. Code Factory is not responsible for damage that might be caused by the software of other companies that can be installed and purchased separately.

If a user wants to test the Software, he/she should ask for a trial license. Under no circumstances will the money paid to purchase licenses be returned, in part or whole. Likewise, under no circumstances will purchased licenses be exchanged for licenses of other Code Factory products, individual components (including but not limited to text-to-speech voices and plug-ins), and separately licensed add-ons.

Each license granted allows installation and activation of the Software on only one phone number, and its associated device, at a time. If you want to use a fully activated version of the Software on several devices at the same time, you must purchase as many licenses as there are devices you wish to use simultaneously. Some of the functionality of the Software may be affected by differences in the firmware of certain devices. Code Factory is not responsible for the failure of the Software due to these factors.

The user declares that he/she has read the content of the website http://www.codefactory.es and other websites it links to, and therefore, has knowledge of the features of the Software. He also declares that he/she has read the Documentation before using the Software and, consequently, knows how to use the software.

7. LIMITATION OF LIABILITY.

Under no circumstances and under no legal theory, tort, contract, or otherwise, shall Code Factory or its dealers and resellers be liable to you or any other person for any damages including, without limitation, indirect, special, incidental, or consequential damages of any character including, without limitation, damages for loss of goodwill, work stoppage, computer failure or malfunction, or any and all other commercial damages or losses, even if Code Factory shall have been informed of the possibility of such damages, or for any claim by any other party. Some states do not allow the exclusion or limitation of incidental or consequential damages, so this limitation and exclusion may not apply to you. Code Factory shall not be liable for any damages under this agreement.

8. U.S. GOVERNMENT RESTRICTED RIGHTS.

If the Software is acquired under the terms of a GSA contract, use, reproduction or disclosure is subject to the restrictions set forth in the applicable ADP Schedule contract. If the Software is acquired under the terms of a DOD or civilian agency contract, use, duplication or disclosure by the government is subject to the restrictions of this license in accordance with 48 C.F.R. 12.212 of the Federal Acquisition Regulations and its successors and 48 C.F.R. 227.7202-1 of the DOD FAR Supplement and its successors.

9. EMERGENCY CALLS.

The devices on which the Software operates use radio signals, wireless, and landline networks as well as user-programmed functions. Because of this, connections in all conditions cannot be guaranteed.

Therefore, you should never rely solely upon any wireless phone for essential communications (for example, medical emergencies). Emergency calls may not be possible on all wireless phone networks or when certain network services and/or phone features are in use. Check with local cellular service providers to determine the methods for placing emergency calls.

To make an emergency call:

A) if the phone is not turned on, switch it on. Check for adequate signal strength. Some networks may require that a valid SIM card be properly inserted in the phone.

B) press the hang up key as many times as necessary (for example, to exit a call, to exit a menu, etc.) in order to clear the display and prepare the phone for calls.

C) key in the emergency number for your present location (for example, 911 or other official emergency number). Emergency numbers vary by location.

D) press the dial key. If certain features are in use, you may first need to turn those features off before you can make an emergency call.

Consult your local cellular service provider for more information.

When making an emergency call, remember to give all the necessary information as accurately as possible. Keep in mind that your wireless phone may be the only means of communication at the scene of an accident – do not end the call until given permission to do so.

10. INTELLECTUAL PROPERTY.

According to the regulations in force referring to royalties, especially the revised text of “Ley Española de Propiedad Intelectual” approved by “Real Decreto Legislativo 1/1996” of 12th April, in Articles 1, 5.2, 8, 95 and further concurrent ones, and as per the international agreements on this matter, in particular the Bern Agreement as well as the Geneva Convention and Council Management of European Communities of 14th May 1991 about legal protection of computer programs, Code Factory, S.L. is the sole owner of the intellectual property rights arising from Mobile Speak, Mobile Magnifier, Mobile Accessibility, Mobile Daisy Player, Mobile Senior and Mobile Geo. The intellectual property rights arising from Orator are shared between Technologies Humanware Canada, Inc. and Code Factory, S.L. It is strictly forbidden to modify, duplicate and/or distribute either totally or partially the Software or any of its components without the authorization of Code Factory, S.L. Such a practice will constitute a legal crime and will be severely punished according to Spanish and international law.

Code Factory, Mobile Speak, Mobile Magnifier, Mobile Accessibility, Mobile Daisy Player, Mobile Senior, Orator and Mobile Geo names and logos are registered trademarks.

Microsoft, Windows Mobile, Windows Phone, Windows Media and their respective logos are registered trademarks of Microsoft Corporation.

Symbian and the Symbian logo are registered trademarks of Symbian Ltd.

11. TERM AND TERMINATION.

This agreement will terminate automatically if you fail to comply with the limitations described above. On termination, you must (i) discontinue your use of the Software, and (ii) permanently erase or destroy all your copies of the Software and Documentation.

12. MISCELLANEOUS.

The Code Factory products require your phone to be subscribed to a network to be fully functional. If the device is in offline mode, flight mode, not subscribed to a network or without a SIM card (in GSM phones) for more than 3 days, it will stop working, although your phone will be fully operational.

This agreement represents the complete agreement concerning this software between the parties and supersedes all prior agreements and representations between them.

In order to amend this agreement, a written addendum executed by both parties is required.

The acceptance of any purchase made by you is expressly made conditional on your assent to the terms set forth herein, and not those contained in your purchase agreement, if any.

If any provision of this agreement is held to be unenforceable for any reason, such provision shall be reformed only to the extent necessary to make it enforceable.

# Contacting Code Factory

Thank you for choosing Mobile Speak and Mobile Magnifier. We hope you have enjoyed exploring the world of mobile phone devices, and harnessed the full potential of multifunctional, mainstream smartphones.

If you would like more information regarding our products please visit our web site at http://www.codefactory.es.

For support on any of our products, sales questions, and general inquiries please visit our Helpdesk at http://www.codefactory.cat/helpdesk/.

# About Code Factory

Founded in 1998 and headquartered in Terrassa/Barcelona, Spain, Code Factory is the global leader in the development of products designed to eliminate barriers to the accessibility of mobile technology for the blind and visually impaired. Today, Code Factory is the leading provider of screen readers, screen magnifiers, and Braille interfaces for the widest range of mainstream mobile devices including Symbian-based, Windows Mobile-powered and Blackberry Smartphones.

Code Factory's success lies in providing excellent customer support and in responding immediately to the needs of its end users. Among our customers are well-known companies and organizations like ONCE, and carriers such as AT&T, Sprint, Vodafone, Telefónica, SFR, Bouygues Telecom, TMN and Telecom Italia. Our partners enable us to provide excellent text-to-speech, in many languages, for Mobile Speak products and to incorporate support for over twenty wireless Braille devices into our software.

# Appendix A: Complete Command Reference

# Keyboard Commands

Notes

–	The commands will also work with the built-in keyboards of Touch Devices, and when using a Touch device in Keypad mode.

- The default Command key changes depending on the phone model. Please refer to the “Command Key submenu” section in Chapter 7 for this information. It can be configured in Configure Mobile Speak > Options > Command key.

## Default Layout

Command + 0 	Toggle Mute

Command + 0 Long 	Toggle Magnification

Command + 1 	Read Date and Time

Command + 2 	Read Current Focus

Command + 4 	Toggle User Dictionary

Command + 4 Long 	Open User Dictionary

Command + 5 	Toggle Command Layouts

Command + 5 Long 	Command Describer (Keyboard Help) Mode

Command + 7 	Change Punctuation Verbosity

Command + 8 	Spell Message

Command + 9 	Change Keyboard Echo

Command + * 	Speech History

Command + * Long 	Secret Mode Verbosity

Command + # 	Read Screen

Command + Left Soft Key 	Read Status, extra information in some application

Command + Right Soft Key 	Configuration

Command + Up Arrow 	Increase Volume

Command + Up Arrow Long	Read Previous Line (Symbian Only)

Command + Down Arrow 	Decrease Volume

Command + Down Arrow Long	Read Next Line (Symbian Only)

Command + Left Arrow 	Decrease Speech Rate

Command + Left Arrow Long	Read Previous Word (Symbian Only)

Command + Right Arrow 	Increase Speech Rate

Command + Right Arrow Long	Read Next Word (Symbian Only)

Command + Enter / Return 	Read Soft Keys

Command + Dial	Toggle Braille (Symbian Only)

Command + Back 	Toggle Profile

Command + Volume up	Change to previous touch mode (Touch only)

Command + Volume down	Change to next touch mode (Touch only)

## Reading Layout

Command + 0 	Toggle Mute

Command + 0 Long 	Toggle Magnification

Command + 1 	Read Previous Sentence

Command + 1 Long 	Read Date and Time

Command + 2 	Read from Top

Command + 2 Long 	Move to Home

Command + 3 	Next Sentence

Command + 4 	Previous Word

Command + 4 Long 	Open User Dictionary

Command + 5 	Toggle Command Layouts

Command + 5 Long 	Command Describer (Keyboard Help) Mode

Command + 6 	Next Word

Command + 6 Long 	Speak Current Word (Symbian Only)

Command + 7 	Read Previous Paragraph

Command + 7 Long 	Change Punctuation Verbosity

Command + 8 	Read from Current Position

Command + 8 Long 	Move to End

Command + 9 	Read Next Paragraph

Command + 9 Long 	Change Keyboard Echo

Command + * 	Speech History

Command + * Long 	Secret Mode Verbosity

Command + # 	Read Screen

Command + # Long 	Spell Current Word (Symbian Only)

Command + Left Soft Key 	Read Status, extra information in some applications

Command + Right Soft Key 	Configuration

Command + Up Arrow 	Increase Volume

Command + Up Arrow Long	Read Previous Line (Symbian Only)

Command + Down Arrow 	Decrease Volume

Command + Down Arrow Long	Read Next Line (Symbian Only)

Command + Left Arrow 	Decrease Speech Rate

Command + Left Arrow Long	Read Previous Word (Symbian Only)

Command + Right Arrow 	Increase Speech Rate

Command + Right Arrow Long	Read Next Word (Symbian Only)

Command + Enter / Return 	Read Soft Keys

Command + Dial	Toggle Braille (Symbian Only)

Command + Back 	Toggle Profile

Command + Volume up	Change to previous touch mode (Touch only)

Command + Volume down	Change to next touch mode (Touch only)

## Magnification Layout

Command + 0 	Toggle Mute

Command + 0 Long 	Toggle Magnification

Command + 1 	Read Date and Time

Command + 2 	Window Title

Command + 3 	Toggle Auto Pan

Command + 4 	Toggle Border

Command + 5 	Toggle Command Layouts

Command + 5 Long 	Command Describer (Keyboard Help) Mode

Command + 6 	Toggle Color Scheme

Command + 8 	Go To Soft Keys

Command + 9 	Toggle Font Smoothing

Command + * 	Speech History

Command + * Long 	Secret Mode Verbosity

Command + # 	Read Screen

Command + Left Soft Key 	Read Status, extra information in some application

Command + Right Soft Key 	Configuration

Up Arrow Long	Pan Up

Command + Up Arrow 	Increase Magnification

Command + Up Arrow Long	Read Previous Line (Symbian Only)

Down Arrow Long 	Pan Down

Command + Down Arrow 	Decrease Magnification

Command + Down Arrow Long	Read Next Line (Symbian Only)

Left Arrow Long 	Pan Left

Command + Left Arrow 	Toggle Lens

Command + Left Arrow Long	Read Previous Word (Symbian Only)

Right Arrow Long 	Pan Right

Command + Right Arrow 	Toggle Screen layout

Command + Right Arrow Long	Read Next Word (Symbian Only)

Command + Enter / Return 	Read Soft Keys

Command + Dial	Toggle Braille (Symbian Only)

Command + Back 	Toggle Profile

Command + Volume up	Change to previous touch mode (Touch only)

Command + Volume down	Change to next touch mode (Touch only)

# Touch Commands

## General Layout (Accessed from Quadrant Mode)

Double tap 	Perform Enter / Return Command

Triple tap 	Exit Command Layout

Slide left 	Left Arrow

Slide right 	Right Arrow

Slide down 	Down Arrow

Slide up 	Up Arrow

Slide any direction and hold	Command List

Tap 1 	Go To Layout

Tap 1 and hold 	Menu Key

Tap 2 	Settings Layout

Tap 2 and hold 	Close Application

Tap 4 	Speech Layout

## Go To Layout (Accessed from Quadrant Mode)

Triple tap 	Exit Command Layout

Slide any direction and hold	Command List

Tap 1	Middle Soft Key (where available)

Tap 3 	Right Soft Key

Tap 4 	Left Soft Key

Double tap 3 	Review Cursor

Tap 4 and hold 	Start Virtual Keyboard

## Settings Layout (Accessed from Quadrant Mode)

Triple tap 	Exit Command Layout

Slide left 	Decrease Speech Rate

Slide right 	Increase Speech Rate

Slide up 	Increase Volume

Slide down 	Decrease Volume

Slide any and hold	Command List

Tap 1 	Toggle Mute

Tap 2 	Toggle Braille

Double tap 1	Secret Mode Verbosity

Double tap 2	Change Keyboard Echo

Double tap 3	Configuration

Double tap 4	Change Punctuation Verbosity

## Speak Layout (Accessed from Quadrant Mode)

Triple tap 	Exit Command Layout

Slide any direction and hold	Command List

Tap 1 	Read Soft Keys

Tap 2 	Read Date and Time

Tap 3 	Read Screen

Tap 4 	Spell Message

Double tap 1 	Read Last Message

Double tap 2  	Read Status and extra information in some applications

Double tap 3 	Read Current Focus

## Review Cursor Mode

Slide left 	Previous Character

Slide right 	Next Character

Slide up 	Previous Line

Slide down 	Next Line

Double tap 	Perform Enter / Return Command

Triple tap	Exit Review Cursor

Tap 1 	Previous Word

Tap 2 	Next Word

## Command List Mode (Accessed from Quadrant Mode)

Slide down	Move to Next Command

Slide up	Move to Previous Command

Slide right	Jump forward in the command list by 3 commands

Slide left	Jump backward in the command list by 3 commands

Double tap	Activate the currently selected command

Triple tap	Exit Command List

## Virtual Touch Keyboard (Accessed from Quadrant Mode)

Slide left 	Previous Character

Slide right	Next Character

Slide up	Previous Line

Slide down	Next Line

Tap 2	Simulate Backspace key

Tap 3	Simulate Return key.

Double tap 3*	Set Next Keyboard Layout

Double tap 2*	Set Previous keyboard layout

Double tap 4*	Set Next Language, if more than one language is installed.

Double tap 1*	Set Previous language, if more than one language is installed.

Triple tap	Exit Virtual Keyboard

* After performing these commands, one can use single tap anywhere to keep cycling the command assignment. Note – 5 seconds of inactivity will turn off this mode or one can use triple tap to go back to Virtual Keyboard.

## Stylus Mode

- To turn on Stylus mode, repeatedly press Command key + Volume Up or Command key + Volume Down to move through the list of touch modes until Stylus mode is enabled.

- To return to the original mode, repeat the same process until the desired mode is enabled.

# Appendix B: Braille Commands

# BAUM Conny / VarioConnect / Pronto

Braille Key Assignments

- Dots 1 through 6: Braille input keys labeled from left to right as 3, 2, 1, 4, 5, 6 on the Conny/VarioConnect.

- Dots 1 through 8: Braille input keys labeled from left to right as 7, 3, 2, 1, 4, 5, 6, 8 on the Pronto.

- Joystick up/down/left/right: four directions of the navigation key on the front panel of the Conny/VarioConnect.

- Joystick press: Center of the joystick on the front panel of the Conny/VarioConnect.

- Space: key between Braille input keys 1 and 4 on the Conny/VarioConnect.

- LSpace or RSpace (either can be pressed): the left or right Space key on the Pronto, or the key to the left or right of the navigation key on the Conny/VarioConnect.

- LRSpaces (pressed together to perform commands): keys immediately to the left and to the right of the navigation key of the Conny/VarioConnect or below the Braille keyboard of the Pronto.

- D1 through D6: set of three keys on either end of the Braille display numbered like the dots of a six-dot cell, with D1, D2 and D3 on the left, and D4, D5 and D6 on the right.

- Outer/Inner LTK/RTK: Round thumbkeys on the front panel of the Conny/VarioConnect, with outer LTK and RTK being on the farthest ends.

- Cursor-routing: key above each Braille cell.

| Command | Action |
| --- | --- |
| Dial 1 | Braille dots according to selected translation table |
| Dial 2 | Braille dots according to selected translation table |
| Dial 3 | Braille dots according to selected translation table |
| Dial 4 | Braille dots according to selected translation table |
| Dial 5 | Braille dots according to selected translation table |
| Dial 6 | Braille dots according to selected translation table |
| Dial 7 | Braille dots according to selected translation table |
| Dial 8 | Braille dots according to selected translation table |
| Dial 9 | Braille dots according to selected translation table |
| Dial 0 | Braille dots according to selected translation table |
| Dial * | Braille dots according to selected translation table |
| Dial # | Braille dots according to selected translation table |
| Enter (Accept) | D3 + D6, Space + dots15(e), Joystick Press, Cursor-Routing in Braille Cursor mode |
| Insert space | Space, LRSpaces (8-dot computer Braille input) and LSpace or RSpace (other Braille grades) |
| Insert Paragraph Marker | Space + dots46, LRSpaces + dots46 |
| Insert Braille character with dot 7 | Inner LTK + Braille character |
| Insert Braille character with dot 8 | Inner RTK + Braille character |
| Braille Escape | Space + dots1356(z), LRSpaces + dots1356(z) |
| Braille Backspace | Space + dots12(b), LRSpaces + dots12(b) |
| Clear/Delete | D4 + D5, Space + dots45, LRSpaces + dots45 |
| Braille Pan Left | D2, Outer LTK |
| Braille Pan Right | D5, Outer RTK |
| Previous Character/Grid Item/Tab Sheet | Space + D3, Joystick Left |
| Next Character/Grid Item/Tab Sheet | Space + D6, Joystick Right |
| Previous Line/List Item/Menu Item | Space + D1, Joystick Up |
| Next Line/List Item/Menu Item | Space + D4, Joystick Down |
| Previous Word | Space + dot2, LRSpaces + dot2 |
| Next Word | Space + dot5, LRSpaces + dot5 |
| Start of text (Top of edit control) | D1 + d2 + D3 |
| End of text (Bottom of edit control) | D4 + D5 + D6 |
| Left Soft key | D2 + D3, LRSpaces + dot3, LSpace or RSpace + Outer LTK |
| Right Soft key | D5 + D6, LRSpaces + dot 6, LSpace or RSpace + Outer RTK |
| Dial/Send | D1 + D3 |
| Hang-Up/End Call | D4 + D6 |
| Applications Menu | D1 + D3 + D4, Space + dots134(m) |
| List of Open Applications | D1 + D3 + D5, Space + dots135(o), LRSpaces + dots135(o) |
| Quick List | D1 + D2 + D3 + D4 + D5, LRSpaces + dots12345(q) |
| Read Soft keys | D2 + D3 + D5 + D6, LRSpaces + dots36 |
| Read Status Information | D2 + D3 + D4, Space + dots234(s), LRSpaces + dots234(s) |
| Open Configure Mobile Speak screen | D1 + D4, Space + dots14(c), LRSpaces + dots14(c) |
| Change User Profile | Space + dots1234(p), LRSpaces + dots1234(p) |
| Press * key in whichever Braille grade (to open Symbols list or unlock keypad) | Space + dots35, LRSpaces + dots35 |
| Repeat Last Spoken Text | D1 + D2 + D3 + D5, Space + dots1235(r), LRSpaces + dots1235(r) |
| Decrease Volume | Space + dots236, LRSpaces + dots236 |
| Increase Volume | Space + dots356, LRSpaces + dots356 |
| Decrease Speed | Space + dots126, LRSpaces + dots126 |
| Increase Speed | Space + 345, LRSpaces + dots345 |
| Change Punctuation Level | D1+D2+D3+D4 |
| Change Keyboard Echo | D1 + D5 |
| Read Date and Time | D1 + D4 + D5, Space + dots145(d), LRSpaces + dots145(d) |
| Toggle Speech Mute | D1 + D3 + D6 |
| Interrupt/Stop Speech | D2 + D5, Space + dots25, LRSpaces + dots25 |
| Toggle Command Help Mode | D1 + D2 + D5, LRSpaces + dots125(h) |
| Change Braille Input Grade | D1 + D2 + D4 + D5, Space + dots1245(g), LRSpaces + dots1245(g) |
| Toggle Braille Cursor | D1 + D2 + D3 + D4 + D5 + D6, LRSpaces + dots123456 |
| Route System Focus to Braille Cursor | Cursor-Routing |

# BAUM SuperVario / HumanWare Brailliant

Braille Key Assignments

- D1 through D6: set of three keys on either end of the Braille display numbered like the dots of a six-dot cell, with D1, D2 and D3 on the left, and D4, D5 and D6 on the right.

- Cursor-routing: key above each Braille cell.

| Command | Action |
| --- | --- |
| Dial 1 | D1 + D6 |
| Dial 2 | D1 + D2 + D6 |
| Dial 3 | D1 + D4 + D6 |
| Dial 4 | D1 + D4 + D5 + D6 |
| Dial 5 | D1 + D5 + D6 |
| Dial 6 | D1 + D2 + D4 + D6 |
| Dial 7 | D1 + D2 + D4 + D5 + D6 |
| Dial 8 | D1 + D2 + D5 + D6 |
| Dial 9 | D2 + D4 + D6 |
| Dial 0 | D3 + D4 + D6 |
| Dial * | D3 + D5 |
| Dial # | D3 + D4 + D5 + D6 |
| Enter (Accept) | D3 + D6, Cursor-Routing in Braille Cursor mode |
| Clear/Delete | D4 + D5 |
| Braille Pan Left | D2 |
| Braille Pan Right | D5 |
| Previous Character/Grid Item/Tab Sheet | D3 |
| Next Character/Grid Item/Tab Sheet | D6 |
| Previous Line/List Item/Menu Item | D1 |
| Next Line/List Item/Menu Item | D4 |
| Previous Word | D2 + D3 + D5 |
| Next Word | D2 + D5 + D6 |
| Start of text (Top of edit control) | D1 + d2 + D3 |
| End of text (Bottom of edit control) | D4 + D5 + D6 |
| Left Soft key | D2 + D3 |
| Right Soft key | D5 + D6 |
| Dial/Send | D1 + D3 |
| Hang-Up/End Call | D4 + D6 |
| Applications Menu | D1 + D3 + D4 |
| List of Open Applications | D1 + D3 + D5 |
| Quick List | D1 + D2 + D3 + D4 + D5 |
| Read Soft keys | D2 + D3 + D5 + D6 |
| Read Status Information | D2 + D3 + D4 |
| Open Configure Mobile Speak screen | D1 + D4 |
| Change User Profile | D1 + D2 + D3 + D4, |
| Press * key in whichever Braille grade (to open Symbols list or unlock keypad) | D3 + D4 |
| Repeat Last Spoken Text | D1 + D2 + D3 + D5 |
| Decrease Volume | D2 + D3 + D6 |
| Increase Volume | D3 + D5 + D6 |
| Decrease Speed | D1 + D2 + D3 + D5 + D6 |
| Increase Speed | D2 + D3 + D4 + D5 + D6 |
| Change Punctuation Level | D1 + D2 + D3 + D4 + D6 |
| Change Keyboard Echo | D1 + D5 |
| Read Date and Time | D1 + D4 + D5 |
| Toggle Speech Mute | D1 + D3 + D6 |
| Interrupt/Stop Speech | D2 + D5 |
| Toggle Command Help Mode | D1 + D2 + D5 |
| Change Braille Input Grade | D1 + D2 + D4 + D5 |
| Toggle Braille Cursor | D1 + D2 + D3 + D4 + D5 + D6 |
| Route System Focus to Braille Cursor | Cursor-Routing |

# HumanWare BrailleConnect Series

Braille Key Assignments

- BK1 through BK6: Braille input keys labeled from left to right as 3, 2, 1, 4, 5, 6.

- Nav up/down/left/right: four directions of the navigation key on the front panel.

- Nav Center: Center of the joystick on the front panel.

- Space: key between Braille input keys 1 and 4.

- LSpace or RSpace (either can be pressed): the key to the left or right of the navigation key on the front panel.

- LRSpaces (pressed together to perform commands): keys immediately to the left and to the right of the navigation key on the front panel.

- D1 through D6: set of three keys on either end of the Braille display numbered like the dots of a six-dot cell, with D1, D2 and D3 on the left, and D4, D5 and D6 on the right.

- Outer/Inner LTK/RTK: Round thumbkeys on the front panel, with outer LTK and RTK being on the farthest ends.

- Touch cursor: key above each Braille cell.

| Command | Action |
| --- | --- |
| Dial 1 | Braille dots according to selected translation table |
| Dial 2 | Braille dots according to selected translation table |
| Dial 3 | Braille dots according to selected translation table |
| Dial 4 | Braille dots according to selected translation table |
| Dial 5 | Braille dots according to selected translation table |
| Dial 6 | Braille dots according to selected translation table |
| Dial 7 | Braille dots according to selected translation table |
| Dial 8 | Braille dots according to selected translation table |
| Dial 9 | Braille dots according to selected translation table |
| Dial 0 | Braille dots according to selected translation table |
| Dial * | Braille dots according to selected translation table |
| Dial # | Braille dots according to selected translation table |
| Enter (Accept) | D4 + D6, Nav Center, Touch Cursor in Braille Cursor mode |
| Insert space | Space, LSpace or RSpace |
| Insert Paragraph Marker | Space + BK4,6, LRSpaces + BK4,6 |
| Insert Braille character with dot 7 | BK7 + Braille character |
| Insert Braille character with dot 8 | BK8 + Braille character |
| Braille Escape | D1 + D3 + D5 + D6, Space + BK1,3,5,6(z), LRSpaces + BK1,3,5,6(z) |
| Braille Backspace | Space + BK1,2(b), LRSpaces + BK1,2(b) |
| Clear/Delete | D4 + D5, Space + dots45, LRSpaces + dots45 |
| Braille Pan Left | D2, Outer Left TK |
| Braille Pan Right | D5, Outer Right TK |
| Previous Character/Grid Item/Tab Sheet | D3, Nav Left |
| Next Character/Grid Item/Tab Sheet | D6, Nav Right |
| Previous Line/List Item/Menu Item | D1, Nav Up |
| Next Line/List Item/Menu Item | D4, Nav Down |
| Previous Word | Space + BK2, LRSpaces + BK2 |
| Next Word | Space + BK5, LRSpaces + BK5 |
| Start of current line (Home) | LRSpaces + BK1,2,3 |
| End of current line (End) | LRSpaces + BK4,5,6 |
| Start of text (Top of edit control) | D1 + d2 + D3 |
| End of text (Bottom of edit control) | D4 + D5 + D6 |
| Left Soft key | LSpace + Outer LTK, RSpace + Outer LTK |
| Right Soft key | LSpace + Outer RTK, RSpace + Outer RTK |
| Dial/Send | D1 + D4 + D5, Inner LTK + Outer LTK |
| Hang-Up/End Call | D1 + D2 + D5, Inner RTK + Outer RTK |
| Applications Menu | D1 + D3 + D4, Space + BK1,3,4(m), LRSpaces + BK1,3,4(m) |
| List of Open Applications | D1 + D3 + D5, Space + BK1,3,5(o), LRSpaces + BK1,3,5(o) |
| Quick List | D1 + D2 + D3 + D4 + D5, LRSpaces + BK1,2,3,4,5(q) |
| Read Soft keys | Outer LTK + Outer RTK |
| Read Status Information | D2 + D3 + D4, Space + BK2,3,4(s), LRSpaces + BK2,3,4(s) |
| Open Configure Mobile Speak screen | Space + BK1,4(c), LRSpaces + BK1,4(c) |
| Change User Profile | Space + Bk1,2,3,4(p), LRSpaces + Bk1,2,3,4(p) |
| Press * key in whichever Braille grade (to open Symbols list or unlock keypad) | D3 + D5, Space + BK3,5, LRSpaces + BK3,5 |
| Repeat Last Spoken Text | D1 + D2 + D3 + D5, Space + BK1,2,3,5(r), LRSpaces + BK1,2,3,5(r) |
| Decrease Volume | Space + BK2,3,6, LRSpaces + BK2,3,6 |
| Increase Volume | Space + BK3,5,6, LRSpaces + BK3,5,6 |
| Decrease Speed | Space + BK1,2,6, LRSpaces + BK1,2,6 |
| Increase Speed | Space + BK3,4,5, LRSpaces + BK3,4,5 |
| Change Punctuation Level | D1 + D2 + D3 + D4 |
| Change Keyboard Echo | D1 + D5 |
| Read Date and Time | Space + BK1,4,5(d), LRSpaces + BK1,4,5(d) |
| Toggle Speech Mute | D1 + D3 + D6 |
| Interrupt/Stop Speech | Inner LTK + Inner RTK |
| Toggle Command Help Mode | LRSpaces + BK1,2,5(h) |
| Change Braille Input Grade | Space + BK1,2,4,5(g), LRSpaces + BK1,2,4,5(g) |
| Toggle Braille Cursor | D1 + D2 + D3 + D4 + D5 + D6, LRSpaces + BK1,2,3,4,5,6 |
| Route System Focus to Braille Cursor | Touch Cursor |

# Optelec EasyLink and EL12

Braille Key Assignments

- Dots 1 through 6: Braille input keys labeled from left to right as 3, 2, 1, 4, 5, 6.

- Space: The middle of three keys in front of the Braille keyboard.

- Shift: Key to the left of the Space key.

- Control: Key to the right of the Space key.

- Left Pan: Round key on the left end of the Braille display of the EL12.

- Right Pan: Round key on the right end of the Braille display of the EL12.

- JUp/Down/Left/Right: Four directions of the joystick between dots 1 and 4 of the EL12.

- JCenter: Center of the joystick between dots 1 and 4 on the EL12.

| Command | Action |
| --- | --- |
| Dial 1 | Braille dots according to selected translation table |
| Dial 2 | Braille dots according to selected translation table |
| Dial 3 | Braille dots according to selected translation table |
| Dial 4 | Braille dots according to selected translation table |
| Dial 5 | Braille dots according to selected translation table |
| Dial 6 | Braille dots according to selected translation table |
| Dial 7 | Braille dots according to selected translation table |
| Dial 8 | Braille dots according to selected translation table |
| Dial 9 | Braille dots according to selected translation table |
| Dial 0 | Braille dots according to selected translation table |
| Dial * | Braille dots according to selected translation table |
| Dial # | Braille dots according to selected translation table |
| Enter (Accept) | JCenter |
| Insert space | Space |
| Insert Paragraph Marker | Space + dots46 |
| Insert Braille character with dot 7 | dot7(Shift) + Braille character |
| Insert Braille character with dot 8 | dot8(Control) + Braille character |
| Braille Escape | Space + dot7(shift) |
| Braille Backspace | Space + dots12(b) |
| Clear/Delete | Space + dots45 |
| Braille Pan Left | Left Pan |
| Braille Pan Right | Right Pan |
| Previous Character/Grid Item/Tab Sheet | JLeft, Space + dot3 |
| Next Character/Grid Item/Tab Sheet | JRight, Space + dot6 |
| Previous Line/List Item/Menu Item | JUp, Space + dot1 |
| Next Line/List Item/Menu Item | JDown, Space + dot4 |
| Previous Word | dot8(Control) + JLeft, Space + dot2 |
| Next Word | dot8(Control) + JRight, Space + dot5 |
| Start of current line (Home) | Space + dot7(Shift) + dot1 |
| End of current line (End) | Space + dot8(Control) + dot4 |
| Start of text (Top of edit control) | Space + dots123 |
| End of text (Bottom of edit control) | Space + dots456 |
| Left Soft key | Space + Left Pan, Space + JLeft -or- Space + dot8(Control) + dot3 |
| Right Soft key | Space + Right Pan, Space + JRight -or- Space + dot8(Control) + dot6 |
| Dial/Send | dot7(Shift) + JCenter |
| Hang-Up/End Call | dot8(Control) + JCenter |
| Applications Menu | Space + dot8(Control), Space + dots134(m) |
| List of Open Applications | Space + dots135(o) |
| Quick List | Space + dots12345(q) |
| Read Soft keys | Space + Left Pan + Right Pan, Space + dots36 |
| Read Status Information | Space + dots234(s) |
| Open Configure Mobile Speak screen | Space + dots14(c) |
| Change User Profile | Space + dots1234(p) |
| Press * key in whichever Braille grade (to open Symbols list or unlock keypad) | Space + dots35 |
| Repeat Last Spoken Text | Space + dots1235(r) |
| Decrease Volume | Space + dots236 |
| Increase Volume | Space + dots356 |
| Decrease Speed | Space + dots126 |
| Increase Speed | Space + dots345 |
| Change Punctuation Level | Space + dot8(Control) + dots1234(p) |
| Change Keyboard Echo | Space + dot8(Control) + dots15(e) |
| Read Date and Time | Space + dots145(d) |
| Toggle Speech Mute | Space + dot8(Control) + dots136(u) |
| Interrupt/Stop Speech | dot8(Control) |
| Change Braille Input Grade | Space + dots1245(g) |
| Route System Focus to Braille Cursor | JCenter, Space + dots15(e) |

# Optelec ALVA Braille Controller 640

Braille Key Assignments

- Up/Down/Left/Right: Four directions of the navigation d-pad in the center of the Smartpad keys below the Braille display.

- Enter: Center of the navigation d-pad.

- SP1 through SP4: Set of two keys on the left and right of the d-pad. SP1 on the farthest left and SP4 on the farthest right have three horizontal and vertical marks respectively, while SP2 on the left and SP3 on the right only have one each. These keys can be pressed quickly or pressed and held for half a second (hold).

- ETTL/BL/TR/BR: Set of two eTouch keys on either end of the Braille display, with three vertical marks on the top-left (TL) and top-right (TR) ETouch keys, while a single horizontal mark is found on the bottom-left (BL) and bottom-right (BR) eTouch keys. These keys can be pressed once (single), twice quickly (double), or pressed and held for half a second (long).

- TK1 through TK5: Thumbkeys on the front panel, numbered from left to right.

- Cursor-routing: Key below each Braille cell.

- Braille FeaturePack: An optional Braille keyboard with keys for Braille input, playing media, etc. can be purchased separately.

| Command | Action |
| --- | --- |
| Dial 1 | Use Braille FeaturePack |
| Dial 2 | Use Braille FeaturePack |
| Dial 3 | Use Braille FeaturePack |
| Dial 4 | Use Braille FeaturePack |
| Dial 5 | Use Braille FeaturePack |
| Dial 6 | Use Braille FeaturePack |
| Dial 7 | Use Braille FeaturePack |
| Dial 8 | Use Braille FeaturePack |
| Dial 9 | Use Braille FeaturePack |
| Dial 0 | Use Braille FeaturePack |
| Dial * | Use Braille FeaturePack |
| Dial # | Use Braille FeaturePack |
| Enter (Accept) | Enter, Cursor-Routing in Braille Cursor mode |
| Insert space | Use Braille FeaturePack |
| Insert Paragraph Marker | Use Braille FeaturePack |
| Insert Braille character with dot 7 | Use Braille FeaturePack |
| Insert Braille character with dot 8 | Use Braille FeaturePack |
| Braille Escape | Use Braille FeaturePack |
| Braille Backspace | Use Braille FeaturePack |
| Clear/Delete | TK3 + SP4 |
| Braille Pan Left | TK1 |
| Braille Pan Right | TK5 |
| Previous Character/Grid Item/Tab Sheet | Left |
| Next Character/Grid Item/Tab Sheet | Right |
| Previous Line/List Item/Menu Item | Up |
| Next Line/List Item/Menu Item | Down |
| Previous Word | TK3 + Left |
| Next Word | TK3 + Right |
| Left Soft key | SP1 |
| Right Soft key | SP4 |
| Dial/Send | SP2 |
| Hang-Up/End Call | SP3 |
| Applications Menu | TK3 + SP1 |
| List of Open Applications | SP1 (hold) |
| Quick List | TK3 + SP2 |
| Read Soft keys | TK3 + Enter |
| Read Status Information | ETTR Single |
| Open Configure Mobile Speak screen | ETTL Long |
| Change User Profile | ETTL Double |
| Press * key in whichever Braille grade (to open Symbols list or unlock keypad) | SP3 (hold) |
| Repeat Last Spoken Text | ETBR Single |
| Decrease Volume | TK3 + ETBR Single |
| Increase Volume | TK3 + ETTR Single |
| Decrease Speed | TK3 + ETBL Single |
| Increase Speed | TK3 + ETTL Single |
| Change Punctuation Level | ETBR Double |
| Change Keyboard Echo | ETBL Double |
| Read Date and Time | ETTL Single |
| Toggle Speech Mute | ETBR Long |
| Interrupt/Stop Speech | TK3 |
| Toggle Command Help Mode | TK3 + ETTL Long |
| Change Braille Input Grade | Use Braille FeaturePack |
| Toggle Braille Cursor | TK2 + TK3 + TK4 |
| Route System Focus to Braille Cursor | Cursor-Routing |
| Disconnect Braille device | SP2 (hold) |

# Eurobraille Esys

Braille Key Assignments

- Dots 1 through 8: Braille input keys labeled from left to right as 7, 3, 2, 1, 4, 5, 6, 8.

- Space: Right of two keys below the keyboard.

- Backspace: Left of two keys below the Braille keyboard.

- LTA/RTA: Left or right triple-action key on either end of the display that can be pressed in three ways (left, right, or center).

- Cursor-routing: Key above each Braille cell.

- JUp/Down/Left/Right: Four directions of the joystick found at either end of the row of cursor-routing keys.

- JCenter: Center of either left or right joystick.

| Command | Action |
| --- | --- |
| Dial 1 | Braille dots according to selected translation table |
| Dial 2 | Braille dots according to selected translation table |
| Dial 3 | Braille dots according to selected translation table |
| Dial 4 | Braille dots according to selected translation table |
| Dial 5 | Braille dots according to selected translation table |
| Dial 6 | Braille dots according to selected translation table |
| Dial 7 | Braille dots according to selected translation table |
| Dial 8 | Braille dots according to selected translation table |
| Dial 9 | Braille dots according to selected translation table |
| Dial 0 | Braille dots according to selected translation table |
| Dial * | Braille dots according to selected translation table |
| Dial # | Braille dots according to selected translation table |
| Enter (Accept) | Right Joystick Center |
| Insert space | Space |
| Insert Paragraph Marker | Space + dots468 |
| Insert Braille character with dot 7 | dot7 + Braille character |
| Insert Braille character with dot 8 | dot8 + Braille character |
| Braille Escape | Space + dots15(e) |
| Braille Backspace | Backspace Key |
| Clear/Delete | Space + dots45 |
| Braille Pan Left | Wheel Left |
| Braille Pan Right | Wheel Right |
| Previous Character/Grid Item/Tab Sheet | Space + dot3 |
| Next Character/Grid Item/Tab Sheet | Space + dot6 |
| Previous Line/List Item/Menu Item | Space + dot1, Right Joystick Up |
| Next Line/List Item/Menu Item | Space + dot4, Right Joystick Down |
| Previous Word | Space + dot2 |
| Next Word | Space + dot5 |
| Start of current line (Home) | Space + dots13 |
| End of current line (End) | Space + dots46 |
| Start of text (Top of edit control) | Space + dots123 |
| End of text (Bottom of edit control) | Space + dots456 |
| Left Soft key | Space + dot7 |
| Right Soft key | Space + dot8 |
| Dial/Send | Backspace + dot7 |
| Hang-Up/End Call | Backspace + dot8 |
| Applications Menu | Space + dots134(m) |
| List of Open Applications | Space + dots135(o) |
| Quick List | Space + dots12345(q) |
| Read Soft keys | Space + dots78 |
| Read Status Information | Space + dots234(s) |
| Open Configure Mobile Speak screen | Space + dots14(c) |
| Change User Profile | Space + dots1234(p) |
| Press * key in whichever Braille grade (to open Symbols list or unlock keypad) | Space + dots35 |
| Repeat Last Spoken Text | Space + dots1235(r) |
| Decrease Volume | Space + dots236 |
| Increase Volume | Space + dots356 |
| Decrease Speed | Space + dots126 |
| Increase Speed | Space + dots345 |
| Change Punctuation Level | Space + dot8 + dots1234(p) |
| Change Keyboard Echo | Space + dot8 + dots15(e) |
| Read Date and Time | Space + dots145(d) |
| Toggle Speech Mute | Backspace + dots136(u) |
| Interrupt/Stop Speech | Backspace + dots78 |
| Toggle Command Help Mode | Space + dots125(h) |
| Change Braille Input Grade | Space + dots1245(g) |
| Toggle Braille Cursor | Space + dots123456 |
| Route System Focus to Braille Cursor | Cursor-Routing |

# Handy Tech Braillino and BrailleStar

Braille Key Assignments

- Dots 1 through 8: Braille input keys labeled from left to right as 7, 3, 2, 1, 4, 5, 6, 8.

- LSpace and RSpace: Left and right Space keys below the Braille keyboard.

- Space: Either the left or right Space key.

- LRSpaces (pressed together to perform commands): Both Space keys.

- LTA/RTA: Left or right triple-action key on either end of the display that can be pressed in three ways ( up, down, or center).

- Cursor-routing: Key above each Braille cell.

| Command | Action |
| --- | --- |
| Dial 1 | Braille dots according to selected translation table |
| Dial 2 | Braille dots according to selected translation table |
| Dial 3 | Braille dots according to selected translation table |
| Dial 4 | Braille dots according to selected translation table |
| Dial 5 | Braille dots according to selected translation table |
| Dial 6 | Braille dots according to selected translation table |
| Dial 7 | Braille dots according to selected translation table |
| Dial 8 | Braille dots according to selected translation table |
| Dial 9 | Braille dots according to selected translation table |
| Dial 0 | Braille dots according to selected translation table |
| Dial * | Braille dots according to selected translation table |
| Dial # | Braille dots according to selected translation table |
| Enter (Accept) | dot 8, RTA Center, Cursor-Routing in Braille Cursor mode |
| Insert space | LSpace or RSpace |
| Insert Paragraph Marker | Space + dots468 |
| Insert Braille character with dot 7 | dot7 + Braille character |
| Insert Braille character with dot 8 | dot8 + Braille character |
| Braille Escape | Space + dots15(e) |
| Braille Backspace | dot 7, Space + dots12(b) |
| Clear/Delete | Space + dots45 |
| Braille Pan Left | RTA Up |
| Braille Pan Right | RTA Down |
| Previous Character/Grid Item/Tab Sheet | Space + dot3 |
| Next Character/Grid Item/Tab Sheet | Space + dot6 |
| Previous Line/List Item/Menu Item | Space + dot1, LTA Up |
| Next Line/List Item/Menu Item | Space + dot4, LTA Down |
| Previous Word | Space + dot2 |
| Next Word | Space + dot5 |
| Start of current line (Home) | Space + dots13 |
| End of current line (End) | Space + dots46 |
| Start of text (Top of edit control) | Space + dots123 |
| End of text (Bottom of edit control) | Space + dots456 |
| Left Soft key | Space + LTA Up, Space + dot7 |
| Right Soft key | Space + RTA Up, Space + dot8 |
| Dial/Send | Space + LTA Down |
| Hang-Up/End Call | Space + RTA Down |
| Applications Menu | LTA Center + RTA Center, Space + dots134(m) |
| List of Open Applications | LRSpaces + RTA Center, Space + dots135(o) |
| Quick List | Space + RTA Center, Space + dots12345(q) |
| Read Soft keys | Space + LTA Up + RTA Up, Space + dots78 |
| Read Status Information | Space + dots234(s) |
| Open Configure Mobile Speak screen | Space + dots14(c) |
| Change User Profile | Space + dots1234(p) |
| Press * key in whichever Braille grade (to open Symbols list or unlock keypad) | Space + dots35 |
| Repeat Last Spoken Text | Space + dots1235(r) |
| Decrease Volume | Space + dots236 |
| Increase Volume | Space + dots356 |
| Decrease Speed | Space + dots126 |
| Increase Speed | Space + dots345 |
| Change Punctuation Level | Space + dot8 + dots1234(p) |
| Change Keyboard Echo | Space + dot8 + dots15(e) |
| Read Date and Time | Space + dots145(d) |
| Toggle Speech Mute | LRSpaces + dots136(u) |
| Interrupt/Stop Speech | LRSpaces |
| Toggle Command Help Mode | Space + dots125(h) |
| Change Braille Input Grade | Space + dots1245(g) |
| Toggle Braille Cursor | Space + dots123456 |
| Route System Focus to Braille Cursor | Cursor-Routing |

# Handy Tech BrailleWave

Braille Key Assignments

- Dots 1 through 8: Braille input keys labeled from left to right as 7, 3, 2, 1, 4, 5, 6, 8.

- Space, Enter, Escape: Three dedicated function keys.

| Command | Action |
| --- | --- |
| Dial 1 | Braille dots according to selected translation table |
| Dial 2 | Braille dots according to selected translation table |
| Dial 3 | Braille dots according to selected translation table |
| Dial 4 | Braille dots according to selected translation table |
| Dial 5 | Braille dots according to selected translation table |
| Dial 6 | Braille dots according to selected translation table |
| Dial 7 | Braille dots according to selected translation table |
| Dial 8 | Braille dots according to selected translation table |
| Dial 9 | Braille dots according to selected translation table |
| Dial 0 | Braille dots according to selected translation table |
| Dial * | Braille dots according to selected translation table |
| Dial # | Braille dots according to selected translation table |
| Enter (Accept) | Enter, Cursor-Routing in Braille Cursor mode |
| Insert space | Space |
| Insert Paragraph Marker | Space + dots468 |
| Insert Braille character with dot 7 | dot7 + Braille character |
| Insert Braille character with dot 8 | dot8 + Braille character |
| Braille Escape | Escape |
| Braille Backspace | Space + dots12(b) |
| Clear/Delete | Space + dots45 |
| Braille Pan Left | dot7 |
| Braille Pan Right | dot8 |
| Previous Character/Grid Item/Tab Sheet | Space + dot3 |
| Next Character/Grid Item/Tab Sheet | Space + dot6 |
| Previous Line/List Item/Menu Item | Space + dot1, LTA Up |
| Next Line/List Item/Menu Item | Space + dot4, LTA Down |
| Previous Word | Space + dot2 |
| Next Word | Space + dot5 |
| Start of current line (Home) | Space + dots13 |
| End of current line (End) | Space + dots46 |
| Start of text (Top of edit control) | Space + dots123 |
| End of text (Bottom of edit control) | Space + dots456 |
| Left Soft key | Space + dot7 |
| Right Soft key | Space + dot8 |
| Dial/Send | Space + dots237 |
| Hang-Up/End Call | Space + dots568 |
| Applications Menu | Space + dots134(m) |
| List of Open Applications | Space + dots135(o) |
| Quick List | Space + dots12345(q) |
| Read Soft keys | Space + dot7 + dots13(k) |
| Read Status Information | Space + dots234(s) |
| Open Configure Mobile Speak screen | Space + dots14(c) |
| Change User Profile | Space + dots1234(p) |
| Press * key in whichever Braille grade (to open Symbols list or unlock keypad) | Space + dots35 |
| Repeat Last Spoken Text | Space + dots1235(r) |
| Decrease Volume | Space + dots236 |
| Increase Volume | Space + dots356 |
| Decrease Speed | Space + dots126 |
| Increase Speed | Space + dots345 |
| Change Punctuation Level | Space + dot8 + dots1234(p) |
| Change Keyboard Echo | Space + dot8 + dots15(e) |
| Read Date and Time | Space + dots145(d) |
| Toggle Speech Mute | dot8 + dots136(u) |
| Interrupt/Stop Speech | Space + dots78 |
| Toggle Command Help Mode | Space + dots125(h) |
| Change Braille Input Grade | Space + dots1245(g) |
| Toggle Braille Cursor | Space + dots123456 |
| Route System Focus to Braille Cursor | Cursor-Routing |

# Handy Tech EasyBraille

Braille Key Assignments

- Dots 1 through 8: Braille input keys labeled from left to right as 7, 3, 2, 1, 4, 5, 6, 8.

- LSpace / RSpace: Left or right Space keys below the Braille keyboard.

- Space: Either left or right space key.

- LRSpaces (pressed together to perform commands): Both Space keys.

| Command | Action |
| --- | --- |
| Dial 1 | Braille dots according to selected translation table |
| Dial 2 | Braille dots according to selected translation table |
| Dial 3 | Braille dots according to selected translation table |
| Dial 4 | Braille dots according to selected translation table |
| Dial 5 | Braille dots according to selected translation table |
| Dial 6 | Braille dots according to selected translation table |
| Dial 7 | Braille dots according to selected translation table |
| Dial 8 | Braille dots according to selected translation table |
| Dial 9 | Braille dots according to selected translation table |
| Dial 0 | Braille dots according to selected translation table |
| Dial * | Braille dots according to selected translation table |
| Dial # | Braille dots according to selected translation table |
| Enter (Accept) | LRSpaces, Cursor-Routing in Braille Cursor mode |
| Insert space | LSpace or RSpace |
| Insert Paragraph Marker | Space + dots468 |
| Insert Braille character with dot 7 | dot7 + Braille character |
| Insert Braille character with dot 8 | dot8 + Braille character |
| Braille Escape | Space + dots1346(x) |
| Braille Backspace | dot 7, Space + dots12(b) |
| Clear/Delete | Space + dots45 |
| Braille Pan Left | Left Space |
| Braille Pan Right | Right Space |
| Previous Character/Grid Item/Tab Sheet | Space + dot3 |
| Next Character/Grid Item/Tab Sheet | Space + dot6 |
| Previous Line/List Item/Menu Item | Space + dot1, LTA Up |
| Next Line/List Item/Menu Item | Space + dot4, LTA Down |
| Previous Word | Space + dot2 |
| Next Word | Space + dot5 |
| Start of current line (Home) | Space + dots13 |
| End of current line (End) | Space + dots46 |
| Start of text (Top of edit control) | Space + dots123 |
| End of text (Bottom of edit control) | Space + dots456 |
| Left Soft key | Space + dot7 |
| Right Soft key | Space + dot8 |
| Dial/Send | LRSpaces + dot7 |
| Hang-Up/End Call | LRSpaces + dot8 |
| Applications Menu | Space + dots134(m) |
| List of Open Applications | Space + dots135(o) |
| Quick List | Space + dots12345(q) |
| Read Soft keys | LRSpaces + dots13(k) |
| Read Status Information | Space + dots234(s) |
| Open Configure Mobile Speak screen | Space + dots14(c) |
| Change User Profile | Space + dots1234(p) |
| Press * key in whichever Braille grade (to open Symbols list or unlock keypad) | Space + dots35 |
| Repeat Last Spoken Text | Space + dots1235(r) |
| Decrease Volume | Space + dots236 |
| Increase Volume | Space + dots356 |
| Decrease Speed | Space + dots126 |
| Increase Speed | Space + dots345 |
| Change Punctuation Level | Space + dot8 + dots1234(p) |
| Change Keyboard Echo | Space + dot8 + dots15(e) |
| Read Date and Time | Space + dots145(d) |
| Toggle Speech Mute | LRSpaces + dots136(u) |
| Interrupt/Stop Speech | LRSpaces |
| Toggle Command Help Mode | Space + dots125(h) |
| Change Braille Input Grade | Space + dots1245(g) |
| Toggle Braille Cursor | Space + dots123456 |
| Route System Focus to Braille Cursor | Cursor-Routing |

# Freedom Scientific Focus Blue

The Freedom Scientific Focus Blue 40 has 40 refreshable Braille cells, A cursor routing button above each Braille cell, A Navrow button above each cursor routing key, 8-key Perkins-style Braille keyboard, with the addition of two SHIFT keys, Two panning buttons, two rocker bars, and two selector buttons, and Two WHIZWHEELS®, one at each end of the display, for quick navigation of files, lists, and menus.

| Command | Action |
| --- | --- |
| Simulate Enter key | key 8 |
| Previous Character/Grid Item/Tab Sheet | Space+dot3 |
| Next Character/Grid Item/Tab Sheet | Space+dot6 |
| Previous Line/List Item/Menu Item | WhizWheel Up or Rocker UP or Space+dot1 |
| Next Line/List Item/Menu Item | WhizWheel down or Rocker down or Space+dot4 |
| Simulate Space key | space |
| Press dot 7 in 8-dot input | key 7 with other Braille keys |
| Press dot 8 in 8-dot input | key 8 with other Braille keys |
| Braille Escape (to clear G1 or G2 text buffer) | key 7 alone or Space + e |
| Braille Backspace | Space+B |
| Delete key | Space+dots45 |
| Insert Paragraph Marker | Space+dots46 |
| Decrease Speed | Space+dots126 |
| Increase Speed | Space+dots345 |
| Braille pan left | Left panning button |
| Braille pan right | right panning button |
| Previous word | Space+dot2 |
| Next word | Space+dot5 |
| Press * key in whichever Braille grade (to open Symbols list or unlock keypad) | Space+dots35 |
| Start of line | panning button + Rocker bar up |
| End of line | panning button + Rocker bar down |
| Top of file or list | Space+dots123 |
| Bottom of file or list | Space+dots456 |
| Read from top of file | Space+dots12356 |
| Read from cursor | Space+dots23456 |
| Activate Left Soft key | Left selector button |
| Activate Right Soft key | Right selector button |
| Dial/Send | Space+dot7+D |
| Hang-Up/End Call | Space+Dot7+H |
| Read status info | Space+S |
| Change profile | Space+P |
| Configure Mobile Speak | Space+C |
| Quick List | Space+Dot7+Q |
| Applications Menu | Space+M |
| Read Soft keys | Left selector button + Right selector button or Space+dots1456 |
| Read date and time | Space+D |
| Repeat Last Spoken Text | Space+r |
| List of Open Applications | Space+dots34 |
| Interrupt speech | Space+dots78 |
| Toggle Braille Cursor | space + 1,2,3,4,5,6 |
| Change Secret Mode | Space+dots16 |
| Help mode on / off? | Space+H |
| Toggle speech mute | Space+U |
| Toggle Keyboard Echo | Space + K |
| Toggle Punctuation | Space+dot7+p |
| Change voice | Space+v |
| Show User Dictionary | Space+dot7+v |
| volume up | Space + dot 236 |
| volume down | Space + dot 356 |
| Change Braille Grade | Space+G |
| Route cursor to cell | Cursor Routing |